3GPP TSG-SA WG2 Meeting #134 	S2-1906972

Sapporo, Japan, June 24-28, 2019 	(revision of S2-19xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1472 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the SMF context transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At SA2#133, in the discussion of the method described in S2-1906744, two ENs were added. According to the first EN, “It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.” The difference from the existing proposal is that the SMF becomes in control of the context transfer instead of the AMF. This could be beneficial in some cases, e.g., when deadlines for context transfer are to be met and the timing information should be managed solely by the Old SMF. It is proposed to update the sequence diagram according to this proposal. The second EN states: “It is FFS if step 6 and 9 can be optimised or changed to a modification of the N4 session”. To change the N4 association from old SMF to new SMF requires updated N4 communication and new UPF functionality. Also, N4 session related state transfer would be needed between old and new SMFs. An alternative solution that offers similar advangates than the above but requires less significant changes is to keep the N4 session (and N4 association) release but add to the N4 session release request from old SMF a re-establishment notification to the UPF. Based on this, the UPF may temporarily keep some information related to the N4 session to optimize packet handling until the N4 session is re-established by the new SMF. Note that this solution can provide backward compatibility for the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to modify the procedure description according to the changes above and remove the current editors notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less optimal solution for SMF context transfer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.26.5	SMF Service Context Transfer procedures

#### 4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

#### 4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_StatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	[Conditional – depending on current subscription] Old SMF subscribes to events when UE status becomes IDLE or RRC inactive (Namf_EventExposure_Subscribe).

3.	[Conditional – depending on the event] The AMF detects the monitored event occurs and sends the event report by means of Namf_EventExposure_Notify message, to the old SMF.

4.	From Old SMF to AMF Nsmf_PDUSession_StatusNotify (SMF transfer indication, New SMF ID.

5.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDUSession_CreateSMContext request (PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication). The same PDU Session ID as received in step 4 is used.

6.	From New SMF to Old SMF SMF Nsmf_PDUSession_ContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

7.	The Old SMF releases the N4 session with the UPF by sending a flag notifying the UPF about the expected re-establishment of the N4 session for the same PDU session. Based on this, the UPF may temporarily keep some information related to the N4 session to optimize packet handling until the N4 session is re-established by the new SMF. All N4 session related information is deleted in the Old SMF.

8.	From Old SMF to New SMF Nsmf_PDUSession_ContextRequest response (SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet as well as the UPF to be selected by the new SMF.

9.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF sets up a new policy association towards PCF.

10.	New SMF performs a full re-establishment of the session (IMS signaling, online charging etc.) and establishes a new N4 session. The parameters from step 8 and, if applies, step 9 are used. All information related to previous N4 session is removed from UPF.

11.	From New SMF to AMF: Nsmf_PDUSession_CreateSMContext response.

12.	New SMF registers to UDM.

13.	New SMF subscribes to subscription changes for the UE.

14.	[Conditional] Old SMF removes its policy association with PCF (triggered by step 7) Any changes to the QoS rules need to be sent to the UE when it becomes active. After Step 13 (Step 7 if there is no policy association) the old SMF can release any internal resources for the PDU session.

*** END CHANGES ***