SA WG2 Meeting #134 	S2-1907302

# June 24-28th, 2019 ; Sapporro, Japan		                                                 (revision of S2-1907302)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0027 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to PDU Session handling for FN-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PDU Session handling for FN-RG requires some clean up e.g. - Referring to BBF/Cable specifications - Avoid repeting information in 23.502 |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDU Session handling for FN-RG is updated to ensure the clean up indicated above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent or sometimes incorrect 23.316 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Basically almost same Tdoc than contributed for SA2 Reno |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.3.4	FN-RG related PDU Session Establishment via W-5GAN

The procedure below is based on the PDU Session Establishment procedure specified in TS 23.502 [3] clause 4.3.2.2.1.

Figure 7.3.4-1: FN-RG related PDU Session Establishment via W-5GAN

1.	After the registration procedure is completed, the W-AGF may establish PDU session(s) on behalf of the FN-RG. The trigger for W-AGF to initiate a PDU establishment process is defined in BBF or cable Specifications.

The W-AGF generates a PDU session ID and derives the parameters for the PDU Session (PDU Session type, S-NSSAI, DNN, SSC mode, etc.) based on signalling received from the FN-RG (DHCP, IPv6 RS, …), on local configuration, and on information received from the 5GC (e.g. during the Registration procedure or when receiving URSP rules) and stored on the W-AGF.

If W-AGF has received a DHCPv4/DHCPv6 request from the FN-RG, it may request a PDU Session with deferred IP address allocation.

The W-AGF sends a NAS PDU Establishment Request to the AMF. This request contains the PDU Session ID, and may contain a Requested PDU Session Type, a Requested SSC mode, 5GSM Capability PCO, SM PDU DN Request Container, Number of Packet Filters.

The W-AGF sends NAS PDU Establishment Request in a N2 Uplink NAS transport message (NAS message, User location information, W-AGF identities).

The W-AGF identities contains a list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF and can be used by SMF in step 8 in TS 23.502 [3] clause 4.3.2.2.1 as input to select an UPF.

2a.	The PDU session request is processed in the 5GC as per steps 2-11 of TS 23.502 [3] clause 4.3.2.2.1. These steps are for UPF selection and resource reservation/allocation in the UPF. With regard to TS 23.502 [3], an additional parameter is sent from AMF to SMF i.e. the list of Identifiers (i.e. FQDN or IP address) of N3 terminations at W-AGF received by the the AMF from the W-AGF. The SMF can use it in step 8 for UPF selection as per clause 4.3.2.2.1.

2b.	The SMF responds via AMF as defined in step 11 of clause 4.3.2.2.1 in TS 23.502 [3] with an N2 PDU Session Request that includes QoS profile(s), PDU Session ID, PDU Session Establishment Accept and the N3 tunnel endpoint information for the UPF. The differences with step 11/12 of TS 23.502 [3] clause 4.3.2.2.1 are

-	N2 SM information does not contain (User Plane Security Enforcement information, UE Integrity Protection Maximum Data Rate, RSN) which do not apply top Wireline access. The W-AGF shall ignore RSN if received from 5GC. The W-AGF should reject N2 SM information that it has received with User Plane Security Enforcement information with a value of Required.

-	N2 SM information does not contain a Session-AMBR but a Session-TMBR,

3.	Based on its own policies, configuration and based on the QoS flows, QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session.

The W-AGF may perform BBF or Cable specific resource reservation with the AN, that is, it sets up the W-UP resources for the PDU session. This step is specified by BBF for W-5GBAN and by CableLabs for W-5GCAN.

4.	The W-AGF allocates AN N3 tunnel information for the PDU Session and includes the AN N3 tunnel endpoint information in the N2 PDU Session Setup Response message to the AMF.

5.	The PDU session setup procedure is completed in 5GC. All steps after step 13 as specified in TS 23.502 [3] figure 4.3.2.2.1 are executed.

6.	If W-AGF requested deferred IP address allocation in step 1a and this was accepted by the network, the W-AGF sends on the user Plane of the PDU Session any DHCP or RS message received beforehand from the FN-RG. If the UE IP address was delivered in NAS in step 2b, the W-AGF provides this IP address to the FN-RG using access network specific means. This allows the IP address allocation to complete with the RG.

NOTE:	the W-AGF can cache a DHCP request received from the FN-RG before this procedure and then send it to the 5GC at step 6 in order to complete the PDU session establishment by an IP address allocation.

NEXT CHANGE

NEXT CHANGE

END OF CHANGES