3GPP TSG-SA WG2 Meeting #134	S2-1907431

# Sapporo, Japan, June 24th – June 28th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0032 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NF Load Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-6-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR addresses two aspects. The first aspect is the insufficient NF load analytics information provided to NF service consumers to select a suitable alternative NF instance in case of overloaded scenario. Insufficient NF load analytics information provided by the NWDAF affects the NF analytics consumers from planning the ongoing services and future service request. TS 23.501 Clause 5.21.3.3 “Reliability of NF instances within the same NF Set” clarifies that, “The NF producer instance is the NF instance which host the NF Service Producer. When the NF producer instance is not available, another NF producer instance within the same NF Set is selected. For Direct Communication mode, the NF Service consumer may subscribe to status change notifications of NF instance from the NRF. If the NF Service consumer is notified by the NRF or detects by itself (e.g. request is not responded) that the NF producer instance is not available anymore, another available NF producer instance within the same NF Set is selected by the NF Service consumer. For Indirect Communication mode, the SCP or NF Service consumer may subscribe to status change notifications of NF instance from the NRF and selects another NF producer instance within the same NF Set if the original NF producer instance serving the UE is not available anymore”. The second aspect the contribution considers is that, even though an “Observation period” indicates the time when the statistics or predictions are requested, there is no means for the NF Service Consumer or the NWDAF to identify the period for which the NF load analytics can be relied upon, i.e. validity period. A prediction information without a validity period will lead to decision making based on an obsolete data at the NF analytics consumer end and so it is clarified to include validity period along with the analytics information during the analytics response and notification from the NWDAF. Further the contribution also add clarity to the target of analytics information by including NF id as given in the figure and procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Analytics filter information with NF Set ID, validity period for the analytics provided by the NWDAF and minor editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF will provide insufficient NFload analytics information to the NF service consumers which affects the alternative NF instance selection in case of overload or failure cases. Further if no validity period provided for an analytics information, NF service consumer will use obsolete analytics information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1, 6.5.2, 6.5.3, 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.5	NF load analytics

6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics Id set to "NF load information";

-	The Target of Analytics Reporting set to NF id;

-	Analytics Filter Information: NF Set ID(s) and an optional list of Instance IDs;

-	An Observation period indicates the time when the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

The NWDAF shall provide the analytics for all NF instances belonging to the specific NF Set. If the NF Instance IDs are provided, then NWDAF shall also provide the analytics for each designated NF instance. In such case the Target of Analytics Reporting should be ignored.

NOTE:	The list of possible instances is limited to AMF, N3IWF, SMF and PCF.

6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1.

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF load | NRF | The load of a specific NF instance(s) in the NF profile as defined per TS.29.510 [18]. |
| NF status | NRF | The status of a specific NF instance(s) as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for a specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7. |

Editor's note:	It is FFS whether the number of registered instances of an NF is needed input data for NWDAF to perform predictions on NF instance load and resource usage.

NOTE:	The OAM information can be used as a complement to NRF information on resources, or as a way to correlate NRF information or as an alternative if NRF information on load is not available.

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition with Table 6.5.2-1.

Table 6.5.2-2: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Load statistics information is defined in Table 6.5.3-1.

Table 6.5.3-1: NF load statistics

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of observed load information for each NF instance(s) along with the corresponding NF id / NF Set ID (as applicable) |
| > Instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the observation period |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |

-	Load predictions information is defined in Table 6.5.3-2.

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of predicted load information for each NF instance |
| > Instance ID | Identification of the NF instance(s) along with the corresponding NF id / NF Set ID (as applicable) |
| > NF status | The availability status of the NF on the Observation period, expressed as a percentage of time |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the observation period |
| > NF peak load (optional) | The maximum load of the NF instance over the observation period |
| > Confidence | Confidence of this prediction |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

6.5.4	Procedures

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to NF load information, the target for analytics is set to NF id. The Analytics Filter Information is set to NF Set ID(s) and an optional list of Instance IDs. The NF can request statistics or predictions or both and can provide a time window.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF resource usage following steps captured in clause 6.2.3.2 for data collection from OAM. Steps 2-5 may be skipped when e.g. the NWDAF already has the requested analytics.

Editor's note:	The exact SA WG5 service used is FFS.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6.	The NWDAF retrieves the NF load and NF status information from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF along with the corresponding Validity Period, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and provide them along with the Validity Period to the NF upon reception of notification of new NF load information from OAM.

* * * * End of Changes * * * *