3GPP TSG-SA WG2 Meeting #134	S2-1907482

# Sapporo, Japan, 24 - 28 June 2019                                            (revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1601 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LLDP support in the UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current text about LLDP support indicates that LLDP needs to be implemented not only in the UPF, but also in the UEs (TT-DS). That would imply an unnecessary implementation burden, given that the LLDP instance in the UPF could also discover the neighbours without any re-implementation of LLDP in the terminal. Furthermore, running LLDP in the terminal would require the definition of a signaling mechanism to transfer topology info from the terminal to the network, which is unnecessary if LLDP is run in the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that LLDP is run in the UPF. Clarify that a single instance of LLDP per logical TSN bridge is sufficient, and the granularity of the TSN bridge is per UPF. Clarify that LLDP in the UPF can be configured according to IEEE specifications, and topology information is reported from UPF to SMF and TSN AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminal side LLDP support puts unnecessary implementation burden on the terminals, and the topology information collection remains unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.2, 5.28.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 4.4.8.2	Architecture to support Time Sensitive Communication

The 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5GS TSN translator functionality consists of Device-side TSN translator (DS-TT) and Network-side TSN translator (NW-TT). 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as (a set of) any other TSN Bridge, the 5GS provides TSN ingress and egress ports via DS-TT and NW-TT. DS-TT and NW-TT optionally supports the following functionality:

-	Hold and forward functionality for the purpose of de-jittering.

Further, the 5GS provides the following functionality on a per logical TSN basis, realized in the UPF:

-	Link layer connectivity discovery and reporting as defined in IEEE 802.1AB [97].

There are three TSN configuration models defined in IEEE P802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this release of the specification;

-	fully distributed model is not supported in this release of the specification;

-	hybrid model is not supported in this Release of the specification.

NOTE 1:	This release only supports interworking with TSN using IEEE 802.1Qbv [96] based QoS scheduling.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

NOTE 2:	Whether DS-TT and UE are combined or are separate is up to implementation.

* * * * Next Change * * * *

### 5.28.3	Port management information exchange in 5GS

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively. Standardized port management information consists of

-	Tx propagation delay, tick granularity and Gate control information (AdminBaseTime, AdminControlList, AdminCycleTime and AdminControlListLength) as defined in IEEE 802.1qbv [96].

NOTE:	AdminCycleTime and AdminControlListLength are optional parameters.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and UPF for LLDP configuration according to IEEE 802.1AB [97]. The information about discovered neighbors shall be reported from UPF to SMF, and further to TSN AF.

Editor's note:	The details of how to transparently convey port management information between TSN AF and DS-TT/NW-TT (including how to determine whether the related port is located in DS-TT or NW-TT) is FFS.

* * * * End of Change * * * *