3GPP TSG-SA WG2 Meeting #134	S2-1907573

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-190XXXX)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1629 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EPS interworking for MA PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EPS interworking is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description on EPS interworking for MA PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS interworking is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.32.7	Interworking with EPS

When a MA PDU Session is established over 3GPP access in 5GC, the MA PDU Sessioin can be transferred to the EPC as a PDN connection in 3GPP access using the EPS interworking procedures in clause 5.17.2. When the MA PDU Session is transferred to the EPC, the ATSSS contexts, e.g. ATSSS rules, measurement assistance information, are released locally by the UE.

When a PDN Connection is established over 3GPP access in EPC, the PDN Connection can be transferred to the 5GS as a PDU Session in 3GPP access using the EPS interworking procedures in clause 5.17.2. The transferred PDU Session can be changed to the MA PDU Session by using PDU Session Modification procedure.

When the network supports N26 interworking, interworking procedures are based on clause 5.17.2.2 with the following modifications:

-	The SMF requests EBI allocation to the AMF if the MA PDU Session is established over 3GPP access and EPS interworking is allowed by subscription and operator policy.

-	When the MA PDU Session is released over 3GPP access, the allocated EBI(s) for the MA PDU Session is released.

-	If the MA PDU Session is established over non-3GPP access, after the MA PDU Session is transferred to the EPC, the SMF triggeres release of the MA PDU Session over non-3GPP access.

-	After a PDN Connection is transferred to the 5GC, if no policy in the UE (e.g. no URSP rule) and no local restrictions mandate a single access for the PDU Session, the UE provides ATSSS Capability to the network by triggering PDU Session Modification procedure. The SMF may change the PDU Session to the MA PDU Session based on ATSSS Capability and local policy.

When the network supports without N26 interworking and the UE supports dual registration mode, interworking procedures are based on clause 5.17.2.3 with the following modifications:

-	If the MA PDU Session is established over non-3GPP access, after the MA PDU Session is transferred to the EPC, the SMF triggeres release of the MA PDU Session over non-3GPP access.

-	When the UE moves a PDN Connection to the 5GC, the UE may include "MA PDU Request" indication and ATSSS Capability during the PDU Session Establishment procedure based on URSP rule.

* * * * End of Changes * * * *