3GPP TSG-SA2 Meeting #134	S2-1907596

# Sapporo, Japan, 24th June 2019 - 28th June 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1615 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502: Resolve EN in Clause 4.22.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ITRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An EN in clause 4.22.7 is left to be resolved: It is FFS if and how the network can request the re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session. It is require a UE to access both 3GPP and Non-3GPP accesses when the UE requests MA PDU Session. In clause 4.2.3.3 has mentioned that “If the UE is in CM-IDLE state in non-3GPP access and if the UE is simultaneously registered over 3GPP and non-3GPP accesses in a PLMN, the network shall initiate a Network Triggered Service Request procedure over 3GPP access.” Therefore this editor’s note can be resolved by the existing mechanism as describe in clause 4.2.3.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove an EN in Clause 4.22.7 by adding a note: Note: The re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session from network can refer to Network Triggered Service Request procedure as describe in clause 4.2.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains unsolve. Also, if we keep the trigger added by corenetwork, this will add unecesary complexity for request reactivation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 1614 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

== First Change ==

### 4.22.7	Adding / Re-activating User-Plane Resources

If the UE has established a MA PDU Session but the user-plane resources over one access of the MA PDU Session have not been established, then:

-	If the UE wants to add user-plane resources over this access, the UE shall initiate the UE Requested PDU Session Establishment procedure over this access, as specified in clause 4.3.2.2. The PDU Session Establishment Request message shall contain the "MA PDU Request" indication and the PDU Session ID of the established MA PDU Session.

-	The PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules for the MA PDU session.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established but are currently inactive (e.g. because the UE is CM-IDLE over this access), then:

-	If the UE wants to re-activate the user-plane resources over this access, then the UE shall initiate the Registration or UE Triggered Service Request procedure over this access, as specified in clause 4.22.9.1 and clause 4.22.8.2, or clause 4.12.4.1.

-	If the network wants to re-activate the user-plane resources over this access and the access is available, then the network shall initiate the Network Triggered Service Request procedure over this access, as specified in clause 4.2.3.3.

Note: The re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session from network can refer to Network Triggered Service Request procedure as describe in clause 4.2.3.3.

== End of Change ==