SA WG2 Meeting #134	S2-1907756

# 24 - 28 June 2019, Sapporo, Japan

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0039 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ## KI #7: automatic GBR QoS restoration at flow setup and handover using V2X Alternative QoS Profile with best effort 5QI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide commonality between URLLC QoS restoration and V2X Uu QoS solutions, an updated solution based on existing solutions #16 and #21 and the V2X solution #27 is described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Combined solution added as a new solution |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GBR machine service not reliable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ********** start of new section ************

## 6.xx	Solution #xx for Key Issue #7: automatic GBR QoS restoration at flow setup and handover using V2X Alternative QoS Profile with best effort 5QI

### 6.xx.1	Description

This is a combination of solutions #16 and #21 and V2X Uu QoS solution#27 (TR 23.786)

Flow Establishment

At QoS Flow establishment, the Core Network signals the Desired QoS Profile and an Alternative QoS Profile with a ‘best effort’ 5QI (e.g. 8 or 9) to the RAN.

The RAN attempts to admit the Desired QoS Profile. If this is not possible, the RAN attempts to admit the Alternative QoS Profile. As the alternative is ‘best effort’ this is likely to succeed. The RAN the informs the Core Network that the Flow has been admitted AND informs the Core Network whether the Desired or Alternative QoS Profile is in use.

If using the Alternative QoS Profile, the RAN then attempts to restore the Flow to its desired QoS profile. The process that the RAN uses is implementation specific but can be ‘event driven’ (e.g. when other UE’s release GFBR flows, or, the UE’s measurement reports indicate that it has returned to better coverage).

If the Desired QoS is restored, the RAN informs the Core Network.

Handover

When the source RAN node attempts handover, it indicates the Desired QoS Profile and the (best effort) Alternative QoS profile to the target RAN node. The target RAN node attempts to admit the Desired QoS profile even if the Alternative QoS proile is currently in use. The Target RAN node informs the source RAN node whether the Desired or the Alternative (best effort) QoS can be “permanently” admitted (or the flow is completely rejected).

If the Target RAN node cannot admit the Desired QoS Profile, the Source RAN node may attempt handover with other Target RAN nodes. Ultimately, the Source RAN node selects one Target RAN node and cancels the handover requests on any other Target RAN nodes.

When the UE completes the handover, the New RAN Node informs the Core Network whether the Desired or Altenative QoS is in use (or if the flow has been released).

If the Alternative QoS is in use on the New RAN node, it attempts to restore the Flow to its desired QoS profile.

### 6.21.2	Procedures

These can be derived from the above description.

### 6.21.3	Impacts on Existing Nodes and Functionality

Common with V2X solution.

### 6.21.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

************ end of changes ********************