SA WG2 Meeting #134 	S2-1907917

# June 24-28th, 2019 ; Sapporro, Japan		                                    (revision of S2-1907792)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0034 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordination between PCF and ACS (for FN RG) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | § 9.6.1 / 9.6.2 / 9.6.3 need cleaning especially about DHCP usage |  |  |  |  |  |  |  |  |
| Summary of change: |  | § 9.6.1 / 9.6.2 / 9.6.3 cleaning especially about DHCP usage (referring to BBF/Cable specifications instead of leaving it unspecified or providing inconsistent information) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC is not aligned with BBF about DHCP usage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.1 ; 9.6.2 ; 9.6.3 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 9.6	Configuration and Management from ACS

### 9.6.1	General

Once the 5G-RG connects to 5GC, the 5G-RG shall establish a PDU session for interaction with the ACS to support the functionalities as described in BBF TR-069 [18] or in BBF TR-369 [19].

Editor's note:	The ACS may also send QoS object to the RG including QoS policy (L2 and L3 marking, see details in BBF TR-198 [30]) to some specific flows. Whether the 5G-RG receives both the QoS object from the ACS and the QoS rule from the 5GC, and the coordination between them are FFS.

### 9.6.2	ACS Discovery

The ACS information may be associated to the RG subscription in the UDM / UDR. In this case the ACS information may be provided to the RG with at least one of the following methods:

-	via DHCP interaction if the RG sends DHCP signalling indicating a request for ACS information. The RG sends a DHCPv4 request including a request for ACS information and receives ACS information from the DHCP server as specified in BBF TR-069 [18] clause 3.1 for ACS Discovery or in BBF TR-369 [19] R-DIS.1 and R-DIS.2.

-	during the PDU session establishment procedure via PCO (protocol Configuration Option) sent in N1 SM message if the 5G-RG has asked to be provided with ACS information via PCO. This applies for 5G-RG only

The ACS information (e.g. URL of the ACS) is defined in BBF TR-069 [18] or in BBF TR-369 [19].

If the RG performs ACS discovery via DHCP process and the SMF is not the DHCP server (e.g. in the case of Ethernet PDU session), the ACS URL is provided by the external DHCP server. In this case, the whole process is transparent to the 5GC and the 5GC is not aware of the ACS information. If the RG performs ACS discovery via DHCP process and the SMF is the DHCP server the ACS information is provided by SMF as part of DHCP process and the SMF shall support the DHCP procedure defined in TR-069 [17] Amendment 6 section 3.2 or in USP (TR-369) [32].

In case the SMF is to provide ACS information to the RG (via PCO or DHCP), it gets this ACS information from SMF subscription data.

The request of ACS information via PCO or via DHCP are mutually exclusive.

The RG may be pre-configured with an ACS information.

The 5G-RG shall consider the ACS information received with the following descending priority order:

1)	ACS information received during the DHCP process.

2)	ACS information received during the PDU session establishment procedure from SMF PCO. This applies for 5G-RG only

3)	The pre-configured ACS information in the RG.

### 9.6.3	ACS Information Configuration by the 3rd party

The ACS information may be configured by a 3rd party AF to the 5GC per subscriber when the SMF is to provide ACS information to the RG.. Subsequently, the ACS discovery via PCO or via DHCP with the DHCP server in the SMF may apply as described in clause 9.6.2.

Figure 9.6.3-1: ACS information configuration procedure

The ACS information configuration procedure enables the 3rd party AF to configure the ACS information  (e.g. URL or IP address) to the 5GC.

1.	The 3rd party AF provides the ACS information  , in the Nnef_ParameterProvision_Update Request to the NEF as in step 1 of TS 23.502 [3] figure 4.15.6.2-1.

2.	As in step 2 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the ACS information  .

3.	As in step 3 and 4 of TS 23.502 [3] figure 4.15.6.2-1 where the provisioned data is the ACS information  .

4.	As in step 5 of TS 23.502 [3] figure 4.15.6.2-1.

5.	As in step 6 of TS 23.502 [3] figure 4.15.6.2-1.

6.	As in step 6 of TS 23.502 [3] figure 4.15.6.2-1 in order to update SMF with with ACS information ;