3GPP TSG-SA2 Meeting #134	S2-1908099

# Sapporo, Japan, 24 - 28th June 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1597 | rev | 1 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of context transfer procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - When to use the push or pull model is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the push or pull model is used by the NF based on specific procedures or implementation. - Other Editorial changes are made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The condition to use push or pull model is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26.1, 4.26.2, 4.26.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

4.26	Network Function/NF Service Context Transfer Procedures

4.26.1	General

Network Function/NF Service Context Transfer Procedures allow transfer of Service Context of a NF/NF Service from a Source NF/NF Service Instance to the Target NF/NF Service Instance, e.g. before the Source NF/NF Service can gracefully close its NF/NF Service. If SMF sets are deployed this applies from an SMF instance within an SMF set to an SMF instance of another SMF set.

The Push and Pull model for context transfer are defined in 4.26.2 and 4.26.3. Which model is used is based on specific procedures or implementation.

4.26.2	NF/NF Service Context Transfer Push Procedure

Figure 4.26.2-1: NF/NF Service Context Push procedure

1.	When triggered, the Source NF/NF Service acting as NF Service Consumer sends its Context (e.g. UE Context or SM context) to the Target NF/NF Service acting as NF Service producer. This may trigger several other procedures that ensure all necessary NF/NF Services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the response indicating the result of the operation (successful or not successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumers of the Source NF/NF Service, which e.g. can be shut down gracefully.

NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

4.26.3	NF/NF Service Context Transfer Pull procedure

Figure 4.26.3-1: NF/NF Service Context Pull procedure

1.	When triggered, the Target NF/NF Service as an NF Service Consumer requests a Context (e.g. UE context or SM context) from the Source NF as a NF Service Producer. This may trigger several other procedures that ensure all necessary NF/NF services are being updated and set up with necessary information about the new context location.

NOTE 1:	Which procedures need to be executed and what information needs to be updated is part of the specfic context transfer procedures as specified in clause 4.26.5.

2.	The NF Service Consumer receives the Context and the operation was successful. When all procedures have been executed successfully the Target NF/NF Service can continue to serve the original NF Service Consumers of the Source NF/NF Service, which e.g. can be shut down gracefully.

NOTE 2:	After resumption of a new service transaction, it may be necessary to contact the UE using existing procedures.

*** END CHANGES ***