3GPP TSG-SA2 Meeting #134	S2-1908091

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0001 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to Observed Service experience related network data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei, Samsung, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Formal inconsistencies between inputs and ouput chapters (6.4.1,6.4.3) - Technical Inconsistencies in input parameters and results - Missing information on predictions; remaining ENs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move the generalities still present in clause 6.4.3 (definition of the analytics, parameters Analytic Filters) to clause 6.4.1 “General”. - Define all the generalites and the requests/subscription parameters in the clause 6.4.1 (as done in other analytics). Besides we classify and rephrase the parameters in accordance with clause 6.1.3 Contents of Analytics Exposure - Specify that analytics are either statistics or predictions, as the main use case is the configuration of future policies for the PCF (=> 2 tables in clause 6.4.3) - Reword some text due to the fact that AF is an NF. - Add the value “any UE” or UE Group as Target of Analytics Reporting in the requests or subscriptions (as a complement to “one or several SUPIs”). Accordingly, the data collection from AFs/NFs is done implicitely with same Target. - Remove redundant information of clause 6.4.1: references to Tables 6.4.1.x. - Update clause 6.4.2 to add AF data collection via NEF. - Remove redundant information of clause 6.4.3: outputs parameters in the procedures, as they are stated in the output tables. - Remove EN: “The differences between procedure 6.4.4.-1 and 6.4.5-1 is FFS”. - Replace "Observation period" by " Analytics target period ". - Remove inappropriate pieces of text related to slicing. - Remove pieces of text related to training a model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Formal and technical inconsistencies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2, 6.4.3, 6.4.4, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## 6.4 Observed Service experience related network data analytics

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS, QoS metrics) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics include:

-	global Service Experience for a Network Slice, given an Application or a set of Applications in a Network Slice, considering all UEs; and

-	detailed Service Experience for a given Application, considering a group of UEs or a list of UEs.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or “any UE”;

-	Analytics Filter Information: one or more combinations of the following Analytics Filters: Area of Interest, Media/application bandwidth, S-NSSAI, DNN, DNAI, Application ID;

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

NOTE:	OAM uses the Observed Service Experience as defined in SA WG5 TSs.

### 6.4.2	Input Data

The service data collected from the AF and the network data from other 5GC NFs for observed service experience are defined in Table 6.4.2-1 and Table 6.4.2-2 here below.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3] ], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | PCF/SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information |  | Provided by the AF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | AMF/PCF/SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

Table 6.4.3-1: Service Experience statistics

| Information | Description |  |
| --- | --- | --- |
| S-NSSAI (optional) |  | Identifies the Network Slice for which analytics information is provided. |  |
| ServiceExperience (1..n) | List of observed service experience information for each Application. |  |
| > Application ID | Identification of the application. |  |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |  |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |  |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |  |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |  |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |  |
| > Validity period | Validity period as defined in clause 6.1.3. |  |
| Slice service experience (optional) |  | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |  |

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI (optional) | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) | List of predicted service experience information for each Application. |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area, where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Areas of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3. |
| > Confidence information | Confidence of this prediction. |
| Slice service experience (optional) | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |

The number of Service Experiences and SUPIs is limited by the maximum number of results provided as input parameter.

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure to NWDAF assisted QoS profile provisioning and adjustment

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE".  At the same time, for an Application Id, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter information = (Application ID, Analytics target periodS-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe  or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [3].

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF offline trains a Service QoE Model for the given application, which will be used to determine/estimate the Service experience for the application later on.

NOTE 1:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

Editor's notes:	The explicit list of input data from OAM (such as PM measurements) is not finalised and FFS.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP

.

NOTE 3:	The call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

If the consumer NF is PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

### 6.4.5	Procedures to request Service Experience reported for UEs on a Network Slice

This procedure is similar to the procedure in clause 6.4.4.

The difference compared to the procedure in clause 6.4.4 is that it allows the consumer to request Analytics ID "Service Experience" for a list of SUPI(s) or Internal Group Identifier(s) .

Therefore, Observed Service experience may be provided in detail, individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

* * * End of changes * * * *