3GPP TSG-SA WG2 Meeting #134	S2-1907058

# Sapporo, Japan, June 24 – 28, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0007 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to QoE calculations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarifying that there exists calculations (in online fashion) and model training (in an offline fashion) for QoE. Also combining two procedures into one, and thereby removing Editor’s Notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Combining two procedures into one, and making small updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes are not removed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3, 6.4.4, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 6.4	Observed Service experience related network data analytics

### 6.4.1	General

NWDAF subscribes the network data from 5GC NF(s) defined in Table 6.4.2-2 and the service data from AF, or via NEF, defined in Table 6.4.2-1 to either calculate service experience (in an online fashion) or train a Service MOS Model (in an offline fashion) for the given application and then provides the observed service experience for the application to a consumer NF or to OAM.

NOTE:	OAM uses the Observed Service Experience as defined in SA WG5 TSs.

The consumer of these analytics shall indicate in the request:

The Analytics ID "Service Experience" with one or multiple combinations of the following Analytics Filters:

-    Online/Offline indication (Indicates either a request for a calculated QoE (in an online fashion) or a request for training a model (in an offline fashion).

-	Application ID.

-	Network area.

-	Time window.

-	Media/application bandwidth.

-	S-NSSAI.

-	DNN.

-	DNAI.

-	    List of SUPIs.

### 6.4.2	Input Data

The service data from the AF and the network data from 5GC NFs for observed service experience are defined in Table 6.4.2-1 and Table 6.4.2-2 here below.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [3].

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | To identify the DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | PCF/SMF | Provided by the AF, which is used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information |  | Provided by the AF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | AMF/PCF/SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

The time stamp are provided by both the AF and each NF to allow correlation of QoS and traffic KPIs for training. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from AF and each NF.

### 6.4.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are used to expose the observed service experience (i.e. average observed Service MoS) from the NWDAF to the consumer 5GC NF (e.g. PCF) or to OAM, indicating e.g. how well the service experience is satisfied by used QoS Parameters and/or how well it satisfy the Service MoS agreed between the MNO and the end user (identified by its SUPI) or between the MNO and the external ASP (identified by the Application ID). A new Analytics ID named "Service Experience" is defined with one or multiple combinations of the following Analytics Filters:

-	Application ID.

-	Network area.

-	Time window.

-	Media/application bandwidth.

-	S-NSSAI.

-	DNN.

-	DNAI.

-	List of SUPIs.

### 6.4.4	Procedures to request Service Experience

Figure 6.4.4-1: Procedure to NWDAF assisted QoS profile provisioning and adjustment

This procedure allows the consumer to request Analytics ID "Service Experience" to either calculate (in an online fashion) or train a model (in an offline fashion) for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates what the target group is, List of SUPIs or "any UE".  If service experience is used to update QoS parameters then, for an Application Id, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Analytics Filter information = (Online/Offline indication, Application ID, time window, SUPI or list of SUPIs, S-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe/ Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID, Target of subscription = Any UE) as defined TS 23.502 [3].

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF will then either calculate QoE or offline train a Service QoE Model for the given application, which will be used to determine/estimate the Service experience for the application later on.

NOTE 1:	The data collection call flow only shows a subscription-notification model for the simplicity instead of both request-response model and subscription-notification model.

Editor's notes:	The explicit list of input data from OAM (such as PM measurements) is not finalised and FFS.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify, indicating how well the calculated service experience or used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP:

a)	The observed Service Experience e.g. average observed Service MOS per application or per slice per user.

b)	Spatial validity condition, where the estimated Service Experience applies. May be left out for Online calculating.

-	If Area of Interest is applied in step 1, spatial validity shall apply to the requested Area of Interest.

c)	Time validity condition, when the estimated Service Experience applies. May be left out for Online calculating.

NOTE 3:	The call flow only shows a request-response model for the simplicity instead of both request-response model and subscription-notification model.

-	If the consumer NF is PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine the used QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

### 6.4.5	Void

* * * End of Changes * * * *