3GPP TSG-SA2 Meeting #134	S2-1907455

# Sapporo, Japan，24 - 28 June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1582 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 Supporting for AF providing UE IP address(es) for 5G VN group PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | External DN may need to assign specific UE IP addresses for 5G VN group PDU sessions. These UE IP addresses can be private IP addresses e.g. 192.168.1.x, which are the same IP address segment as enterprise network. The N6 PtP tunnel can be used to support the PDU packet transmission when the UE’s IP address is private IP address. In R15, the DHCP has been introduced for IP address allocation by external DN.However the UE may need to indicate to the network within the Protocol Configuration Options that the UE requests to obtain the IPv4 address with DHCPv4, which increases the complexity in UE side. And in real network, it is difficult for SMF communicating with external DN’s DHCP server. Therefore, one solution is provided in this paper that the AF may provide specific IP addresses for one 5G VN group members. And during the PDU session establishment, static IP address allocation method can be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AF can provide speicifc UE IP ddresses for 5G VN group PDU sessions, corresponding update in 502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if not approved, 5G VN can not support requirements from enterprise network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

#### 4.15.6.3b	5G VN group data

The 5G VN group data is described in Table 4.15.6.3b-1.

# Table 4.15.6.3b-1: Description of 5G VN group data

| Parameters | Description |
| --- | --- |
| DNN | DNN for the 5G VN group |
| S-NSSAI | S-NSSAI to the 5G VN group's communication session |
| PDU Session Type | PDU Session Types allowed for 5G VN group's communication session |
| Application descriptor | The may be multiple instances of this information; this information may be used to build URSP sent to group members |
| UE IP address(es) | UE IP address(es) corresponding to the 5G VN group members |

The information described in Table 4.15.6.3b-1 corresponds to 5G VN group data that an AF may provide together with External Group ID.

<<< End of changes >>>