3GPP TSG-SA2 Meeting #134	S2-1907523

# Sapporo, Japan, 24 - 28th June 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1598 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF Service Context Transfer procedure update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA-Exception |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Solve the second editor’s note in clause 4.26.5. - The service description for Nsmf_PDUSession_SMContextStatusNotify and Nsmf_PDUSession_ContextPush are not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change Nsmf_PDUSession_StatusNotify to Nsmf_PDUSession_SMContextStatusNotify in clause 4.26.5.2. - In clause 4.26.5.3, (1) In step 2, change Nsmf_PDUSession_StatusNotify to Nsmf_PDUSession_SMContextStatusNotify. (2) For the second editor’s note, it is proposed to delete it and change N4 session establish to N4 session modification and change SM policy association creation to update. (3) In the original figure, step 6 and 13 are removed due to the revision in the above bullet. 3. Update the service description for Nsmf_PDUSession_SMContextStatusNotify 4. Udpate the service description for Nsmf_PDUSession_ContextPush |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The second editor’s note can’t be solved and the SMF context transfer procedure is not optimized. - The service description for Nsmf_PDUSession_SMContextStatusNotify and Nsmf_PDUSession_ContextPush are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.26.5, 5.2.8.2.8, 5.2.8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

4.26.5	SMF Service Context Transfer procedures

4.26.5.1	General

This clause lists the context-specific transfer procedures between different SMF Sets supporting the same DNN/S-NSSAI pair supported for SM Contexts (i.e. SMF contexts).

4.26.5.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_SMContextStatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

4.26.5.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In the case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.26.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

NOTE:	If UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.26.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	From Old SMF to AMF: Nsmf_PDUSession_SMContextStatusNotify (SMF transfer indication, New SMF ID)

3.	The AMF waits until UE becomes IDLE or RRC inactive.

Editors note:	It is FFS if step 2-3 can be replaced with a subscribe notify of the UE status from SMF to AMF.

4.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (SUPI, PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication, old SMF ID). The same PDU Session ID as received in step 1 is used.

5.	From New SMF to Old SMF SMF Nsmf_PDUSession_ContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	From Old SMF to New SMF: Nsmf_PDUSession_ContextRequest response (SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv4v6, or the Ethernet MAC address(es) in case PDU session type Ethernet. The Old SMF can release any internal resources for the PDU session.

7.	[Condtitional] If dynamic PCC is used for the PDU Session, the new SMF  performs SM Policy Association Modification towards PCF

8.	New SMF modifies the N4 session. The new SMF notifies the UPF that it has taken over the responsibility of the signalling path towards the UE. The parameters from step 6 and, if applies, step 7 are used.

9.	From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

10.	New SMF registers to UDM.

11.	New SMF subscribes to subscription changes for the UE

**************** 2nd Change ****************

5.2.8.2.8	Nsmf_PDUSession_SMContextStatusNotify service operation

Service operation name: Nsmf_PDUSession_SMContextStatusNotify.

Description: This service operation is used by the SMF to notify its consumers about the status of an SM context related to a PDU Session (e.g. PDU Session release or SMF context transfer due to local reasons within the SMF, PDU Session handover to a different system or access type). The SMF may use this service operation to update the SMF derived CN assisted RAN parameters tuning in the AMF.

Input, Required: Status information.

Input, Optional: Cause, SMF derived CN assisted RAN parameters tuning, SMF transfer indication, I-SMF transfer indication, New SMF ID.

Output, Required: Result Indication.

Output, Optional: None.

**************** 3rd Change ****************

5.2.8.2.11	Nsmf_PDUSession_ContextPush service operation

Service operation name: Nsmf_PDUSession_ContextPushRequest

Description: This service operation is used by the SMF as Service Consumer to push one SM Context to another SMF as NF Service Producer. It may be triggered by OAM or due to local reasons within the SMF.

Input, Required: SM Context of identified PDU session.

The SM context includes SM context in I-SMF(or V-SMF) and SM context in SMF (or H-SMF) separately.

See Table 5.2.8.2.10-1 for single SM Context stored in I-SMF or V-SMF that may be transferred to another SMF instance.

Editor's note:	The SM context stored in SMF(or H-SMF) is to be defined.

Output, Required: Result Indication.

Output, Optional: Cause.

See clause 4.26.2 for an example usage of this service operation.