3GPP TSG-SA WG2 Meeting #134	S2-1907564

# Sapporo, Japan, June 24 – 28, 2019											  (revision of S2-190XXXX)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1606 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of access addition – Option 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE is registered in both accesses in the same PLMN, during the MA PDU Session Establishment procedure, the SMF tries to establish a user plane resources in both accesses. In this case, if the MA PDU Session is established in home routed mode or I-SMF is inserted, the V-SMF or I-SMF may fail to setup user plane resources due to e.g. UE is CM-IDLE in non-3GPP access or UE is in non-allowed area in 3GPP access. In this case, the UE will send PDU Session Establishment message to add second access. Then (H-)SMF receives PDU Session Establishment message even though assoicated SM contexts are already exist. In order to solve this issue, it is proposed that the SMF provides an information on whether the MA PDU Session is established over a single access or both accesses in PDU Session Establishment Accept message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SMF performs setup of user-plane resources over the access different from the access the PDU Session Establishment request is received before sending PDU Session Accept message to the UE. The SMF provides an information on whether the MA PDU Session is established over a single access or both accesses in PDU Session Establishment Accept message to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF may receive PDU Session Establishment request message when the assoicated SM contexts are already created. It is not clear how the SMF should handle such case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.2.1, 4.22.2.2, 4.22.3, 4.22.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.22.2	UE Requested MA PDU Session Establishment

#### 4.22.2.1	Non-roaming and Roaming with Local Breakout

The signalling flow for a MA PDU Session establishment when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.2.2.1-1 with the following differences and clarifications:

-	The PDU Session Establishment Request message may be sent over the 3GPP access or over the non-3GPP access. In the steps below, it is assumed that it is sent over the 3GPP access.

-	In step 1, the UE provides a "MA PDU Request" indication and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions).

The "MA PDU Request" indication and the Request Type = "initial request" indicate to the network that this PDU Session Establishment Request is to establish a new MA PDU Session and to apply the ATSSS-LL functionality, or the MPTCP functionality, or both functionalities, for steering the traffic of this MA PDU session.

If the UE requests an S-NSSAI and the UE is registered over both accesses, it shall request an S-NSSAI that is allowed on both accesses.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects an SMF, which supports MA PDU sessions.

-	In step 3, the AMF informs the SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication) and, in addition, it indicates to SMF whether the UE is registered over both accesses. If the AMF determines that the UE is registered via both accesses but the requested S-NSSAI is not allowed on both accesses, then the AMF shall reject the MA PDU session establishment.

-	In step 7, the SMF sends an "MA PDU Request" indication to the PCF in the SM Policy Control Create message. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The PCF provides PCC rules for the MA PDU session, i.e. PCC rules that include ATSSS policy control information, as specified in TS 23.503 [20]. From the received PCC rules, the SMF derives (a) ATSSS rules, which will be sent to UE for controlling the traffic steering, switching and splitting in the uplink direction, and (b) N4 rules, which will be sent to UPF for controlling the traffic steering, switching and splitting in the downlink direction. If the UE indicates the support of "ATSSS-LL Capability", the SMF may derive the Measurement Assistance Information.

-	If the SMF was not informed in step 2 that the UE is registered over both accesses, in the remaining steps of Figure 4.3.2.2.1-1, the SMF establishes the user-plane resources over the 3GPP access, i.e. over the access where the PDU Session Establishment Request was sent on:

-	In step 10, the N4 rules derived by SMF for the MA PDU session are sent to UPF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include information for measurement into the N4 rule to instruct the UPF to initiate performance measurement for this MA PDU Session. In step 10a, the UPF allocates addressing information for the Performance Measurement Function (PMF) in the UPF. In step 10b, the UPF sends the addressing information for the PMF in the UPF to the SMF.

-	In step 13, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by SMF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include the addressing information of PMF in the UPF into the Measurement Assistance Information.

-	If the SMF was informed in step 2 that the UE is registered over both accesses, the SMF initiates the establishment of user-plane resources as follows:

-	The SMF establishes user-plane resources over non-3GPP access first as described in Figure 4.3.2.2.1-1 with following differences:

-	In step 10, the N4 rules derived by SMF for the MA PDU session are sent to UPF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include information for measurement into the N4 rule to instruct the UPF to initiate performance measurement for this MA PDU Session. In step 10a, the UPF allocates addressing information for the Performance Measurement Function (PMF) in the UPF. In step 10b, the UPF sends the addressing information for the PMF in the UPF to the SMF.

-	The SMF sends an N1N2 Message Transfer to AMF including N2 SM Information and indicates to AMF that the N2 SM Information should be sent over non-3GPP access. The N1N2 Message Transfer does not include an N1 SM Container for the UE.

-	After the user-plane establishment is finished over non-3GPP access, the SMF initiates the establishment of user-plane resources over 3GPP access as described in Figure 4.3.2.2.1-1 with following differences:

-	In step 13, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes an information on whether the MA PDU Session is established over a single access or both accesses and the ATSSS rules for the MA PDU session, which were derived by SMF. If the ATSSS Capability for the MA PDU Session indicates "ATSSS-LL Capability", the SMF may include the addressing information of PMF in the UPF into the Measurement Assistance Information.

How user-plane resources can be added over an access of the MA PDU Session is specified in clause 4.22.7 below.

#### 4.22.2.2	Home-routed Roaming

When the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the differences and clarifications:

-	In step 1, the UE provides a "MA PDU Request" indication and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions).

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF and an H-SMF, which supports MA PDU sessions.

-	In step 3, the AMF also includes an "MA PDU Request" indication.

-	In step 6, the V-SMF also includes an "MA PDU Request" indication.

-	In step 9, the H-SMF sends an "MA PDU Request" indication to H-PCF in the SM Policy Control Create message. The H-PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

The H-PCF provides the PCC rules for the MA PDU session and the H-SMF derives the ATSSS rules for the UE and the N4 rules for the H-UPF.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes an information on whether the MA PDU Session is established over a single access or both accesses and the ATSSS rules for the MA PDU session, which were derived by H-SMF, and may include Measurement Assistance Information.

When the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the MA PDU Session is established as specified in Figure 4.3.2.2.2-1 ("UE-requested PDU Session Establishment for home-routed roaming scenarios") with the following differences and clarifications:

-	In step 1, the UE provides a "MA PDU Request" indication and an ATSSS Capability (e.g. an "MPTCP Capability" and/or an "ATSSS-LL Capability"), as defined in TS 23.501 [2], clause 5.32.2 (Multi Access PDU Sessions).

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects a V-SMF, which supports MA PDU sessions.

-	In step 3, the AMF informs the V-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication).

-	In step 6, the V-SMF informs the H-SMF that the request is for a MA PDU Session (i.e. it includes an "MA PDU Request" indication).

-	In step 9, the H-SMF sends an "MA PDU Request" indication to PCF in the SM Policy Control Create message. The PCF decides whether the MA PDU session is allowed or not based on operator policy and subscription data.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which indicates to UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU session, which were derived by H-SMF, and may include Measurement Assistance Information.

-	After the successful completion of the procedure in Figure 4.3.2.2.2-1, i.e. after the MA PDU Session is successfully established on the first access, the UE shall initiate again the procedure in Figure 4.3.2.2.2-1 over the other access. In particular:

-	In step 1, the UE shall send another PDU Session Establishment Request over the other access containing also a "MA PDU Request" indication and the same PDU Session ID that was provided over the first access.

-	In step 16, the UE receives another PDU Session Establishment Accept message, which may contain updated ATSSS rules for the MA PDU session.

* * * * Start of 2nd Change * * * *

4.22.3	UE Requested PDU Session Establishment with Network Modification to MA PDU Session

When an ATSSS-capable UE requests to establish a single-access PDU Session, but no policy in the UE and no local restrictions mandate a single access, the 5GC network may decide to modify it to a Multi-Access PDU (MA PDU) Session. This decision may be taken when e.g. the SMF wants to offload some traffic of the requested PDU Session to non-3GPP access or when the SMF wants to apply MPTCP to provide bandwidth aggregation for the requested PDU Session.

In the case of non-roaming or roaming with local breakout, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session is the same with the procedure specified in clause 4.22.2.1, with the following clarifications and modifications:

-	In step 1, the UE does not include the "MA PDU Request" indication but it may include the "MA PDU Network-Upgrade Allowed" indication and its ATSSS Capability, if no policy in the UE (e.g. no URSP rule) and no local restrictions mandate a single access for the requested PDU Session. The "MA PDU Network-Upgrade Allowed" indication indicates that the requested single-access PDU Session may be converted to a MA PDU Session, if the 5GC network wants to.

-	In step 2, if the AMF receives the "MA PDU Network-Upgrade Allowed" indication, the AMF may select a SMF that supports MA PDU sessions. The AMF does not send the "MA PDU Request" indication to SMF, but it may send the "MA PDU Network-Upgrade Allowed" indication, if received from the UE. If the AMF sends the "MA PDU Network-Upgrade Allowed" indication to SMF, it shall also indicate to SMF whether the UE is registered over both accesses. In this step, the SMF is informed of whether the UE can support ATSSS and whether it is registered on both accesses, or on one access only.

-	After step 6, if SMF receives the "MA PDU Network-Upgrade Allowed" indication, the SMF may decide to convert the single-access PDU Session requested by the UE into a MA PDU Session. The SMF may take this decision based on local operator policy and/or other conditions, which are not specified in the present document.

-	In step 7, the SMF indicates to PCF that the SM policy control information is requested for a MA PDU Session.

-	If the SMF was informed in step 2 that the UE is registered over both accesses, the SMF triggers the establishment of user-plane resources in both accesses as described in clause 4.22.2.1.

-	The PDU Session Establishment Accept message includes an information on whether the MA PDU Session is established over a single access or both accesses and ATSSS rules which indicate to UE that the requested PDU Session was converted by the network to a MA PDU Session.

In the case of home-routed roaming, when the UE is registered to the same VPLMN over 3GPP access and non-3GPP access, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session but no policy in the UE and no local restrictions mandate a single access, is the same with the procedure specified above.

In the case of home-routed roaming, when the UE is registered to different PLMNs over 3GPP access and non-3GPP access, the procedure for establishing a MA PDU Session when the UE requests a single-access PDU Session but no policy in the UE and no local restrictions mandate a single access, is the same with the procedure specified in clause 4.22.2.2, with the following clarifications and modifications:

-	In step 1, the UE does not include the "MA PDU Request" indication but it may include an "MA PDU Network-Upgrade Allowed" indication and its ATSSS Capability (e.g. the "ATSSS-LL Capability" and/or the "MPTCP Capability"). The UE may include the "MA PDU Network-Upgrade Allowed" indication and its ATSSS Capability as defined above.

-	In step 2, if the AMF receives the "MA PDU Network-Upgrade Allowed" indication, the AMF may select a V-SMF that supports MA PDU sessions. The AMF does not send the "MA PDU Request" indication to V-SMF, but it may send the "MA PDU Network-Upgrade Allowed" indication, if received from the UE.

-	In step 6, the V-SMF does not provide the "MA PDU Request" indication to H-SMF, but it provides the "MA PDU Network-Upgrade Allowed" indication, if received from AMF.

-	After step 6, if the H-SMF receives the "MA PDU Network-Upgrade Allowed" indication, the H-SMF may decide to convert the single-access PDU Session requested by the UE into a MA PDU Session. The H-SMF may take this decision based on local operator policy and/or other conditions, which are not specified in the present document.

-	In step 16, the UE receives a PDU Session Establishment Accept message, which includes ATSSS rules and indicates to UE that the requested single-access PDU session was established as a MA PDU Session.

-	After the MA PDU Session is established over one access, the UE shall send another PDU Session Establishment Request over the other access containing a "MA PDU Request" indication and the same PDU Session ID that was provided over the first access.

* * * * Start of 3rd Change * * * *

4.22.7	Adding / Re-activating User-Plane Resources

If the UE has established a MA PDU Session but the user-plane resources over one access of the MA PDU Session have not been established, then considering the information on whether the MA PDU Session is established in both accesses in the PDU Session Establishment Accept message:

-	If the UE wants to add user-plane resources over this access, the UE shall initiate the UE Requested PDU Session Establishment procedure over this access, as specified in clause 4.3.2.2. The PDU Session Establishment Request message shall contain the "MA PDU Request" indication and the PDU Session ID of the established MA PDU Session.

-	The PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules for the MA PDU session.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established but are currently inactive (e.g. because the UE is CM-IDLE over this access), then:

-	If the UE wants to re-activate the user-plane resources over this access, then the UE shall initiate the Registration or UE Triggered Service Request procedure over this access, as specified in clause 4.22.9.1 and clause 4.22.8.2, or clause 4.12.4.1.

-	If the network wants to re-activate the user-plane resources over this access and the access is available, then the network shall initiate the Network Triggered Service Request procedure over this access, as specified in clause 4.2.3.3.

Editor's note:	It is FFS if and how the network can request the re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session.

* * * * End of Changes * * * *