SA WG2 Meeting #134 	S2-1907853

# June 24-28th, 2019 ; Sapporro, Japan		                                    (revision of S2-1907379)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0030 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of IPTV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description of IPTV support in 23.316 required improvements such as listed in the 5WWC exception sheet SP-190437. Furthermore IPTV is to be supported also over Wireline access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the specification of N7 (Npcf-sm …) and N4 about the impacts of IPTV Adding clarifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IPTV cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1 ; 9.3 ; 4.6.3 ; 4.6.4 ; 4.6.X ; 9.X ; 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Tagged in yellow: Marco Tagged in grey: further added by LTH Tagged in blue : indication of further modifications to be done Preview is controlled by the PCF |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 7.7.1	IPTV

#### 7.7.1.1	Overview

In this Release, in order to support IPTV services, following principles apply:

-	the 5G-RG supports IP PDU Session Type

-	IP multicast traffic received from N6 interface is replicated by UPF and sent over PDU Sessions

-	IGMP messages from the STB or from the 5G-RG are terminated and managed by the UPF acting as PSA

NOTE: whether  IGMP terminates in the 5G RG or in another entity (e.g. STB) is out of the scope of 3GPP.

- The SMF controls the support of IPTV by the UPF acting as PSA using PDR, FAR, QER, URR. This includes control of which IGMP requests the UPF is to accept.

marco’s comments: please add the others  principles that we agreed

This clause describes the procedures that support IPTV in 5G system including the procedures below:

Registration and PDU Session Establishment procedure for IPTV is shown in clause 7.7.1.1.1. The Registration Procedure is used to register to 5GS and the PDU Session Establishment Procedure is used to establish the PDU Session used for IPTV Service.

-	IPTV Access procedure shown in clause 7.7.1.1.2 may, depending on the deployment, be used to access the IPTV network, e.g. completing the IPTV Authentication and IP allocation.

-	Unicast/Multicast Packets transmission procedure shown in clause 7.7.1.1.3. The procedure specifies how to transmit unicast/multicast packets related with IPTV service over 5GC.

In this release of the specification, the 5GC does not assume any traffic replication capability in the 5G AN (NG-RAN or W-5GAN).

##### 7.7.1.1.1	Registration and PDU Session Establishment procedure for IPTV

5G-RG perform Registration procedure described in clause 4.2.2.2.2 of TS 23.502 [3] with the following differences:

-	UE is replaced by 5G-RG.

5G-RG perform PDU Session establishment procedure described in clause 4.3.2.2.1 of TS 23.502 [3] applies with the following differences and clarifications:

-	UE is replaced by 5G-RG.

-	In step 1 of TS 23.502 [3] clause 4.3.2.2.1, 5G-RG may indicate within the Protocol Configuration Options element that the UE requests to obtain the IPv4 address with DHCPv4.

-	5G-RG shall establish an IP-based PDU Session with a specific (DNN, S-NSSAI) for IPTV service.

-	In step 7b and 9 of clause 4.3.2.2.1 of TS 23.502 [3], the PCF provides PCC Rules including information related to IPTV Service. This is specified in clause 9.3.1.

-	The SMF sends to the UPF acting as PSA N4 rules such as PDR, FAR related to IP Multicast traffic allowed for the PDU Session. This may take place at steps steps 10a and 16a of TS 23.502 [3] clause 4.3.2.2.1-1. Such N4 rules are further described in clause 4.6.3, 4.6.4 and 4.6.5. IP Multicast traffic allowed for the PDU Session corresponds to IPTV services allowed for the user.

NOTE:	The interactions between STB and 5G-RG are specified in TR-124 [5] in BBF and not shown in this clause.

##### 7.7.1.1.2	IPTV Access procedure

In case of IPTV network access control based on the DHCP procedure, 5G-RG may be configured to retrieve via DHCP the IP address that it will use to access IPTV services. The DHCP procedure described in TS 23.501 [2] clause 5.8.2.2 is carried out with the difference shown below:

-	When the SMF receives the Uplink DHCP message, the SMF may be configured to insert the IPTV access control information as received in subscription data from UDM to the uplink DHCP message.

NOTE 1:	The IPTV access control information can include a line ID defined in RFC 3046 [20] or any other identity which can be used to identify the IPTV subscriber. This is based on IPTV deployment and 3GPP doesn't define the the IPTV access control information that the SMF copies from subscription data to DHCP signalling.

NOTE 2:	The interactions between STB and 5G-RG is  specified in BBF TR-124 [5] and not described in this procedure.

NOTE 3:	The description of interactions among the elements part of the IPTV network is out of 3GPP scope.

##### 7.7.1.1.3	Unicast/Multicast Packets transmission procedure

5GS can support Unicast Service from IPTV network directly.

In order to obtain the multicast service from IPTV network, the Multicast Packets transmission procedure should be performed. The procedure in figure 7.7.1.1-3 describes how the 5G-RG joins an IP multicast group.

Figure 7.7.1.1-3: 5G-RG join IP Multicast Packets transmission procedure

1.	The 5G-RG send an IGMP Join via the IP PDU Sesssion user plane.

2.	When UPF detects the IGMP Join, the UPF may identify the IGMP Join packets based on PDR received over N4 as described in clause 4.6.3 and handle the IGMP Join accordingly based on FAR as described in clause 4.6.4. An example is given as below:

-	If the IP Multicast Addressing information included in the IGMP Join message is allowed to be accessed via the PDU Session the UPF shall add the PDU Session to the requested multicast group. If requested by an URR, the UPF notifies the SMF that the UE is joining to a multicast group, providing the associated IP Multicast Addressing information.

-	If the IP Multicast Addressing information included in the IGMP Join message is not allowed to be accessed via the PDU Session, the UPF shall drop the IGMP Join message.

.

The UPF acts as a Multicast Router as defined in IETF RFC 2236: "Internet Group Management Protocol, Version 2 [xx] and IETF RFC 3376 [28]: "Internet Group Management Protocol, Version 3". This may include following actions:

-	if the IGMP Join message is the first IGMP request the UPF has received about the target IP multicast traffic (IP Multicast Addressing information): the UPF exchanges N6 signalling such as PIM (Protocol-Independent Multicast) in order to connect to the N6 multicast distribution tree related with this IP multicast traffic; This ensures that the UPF receives the DL multicast traffic.

-	The IP multicast related signalling protocol used on N6 (e.g. Sparse Mode PIM-SM) to be supported over N6 is defined by local policies on the UPF.

2b.	if the SMF had set the corresponding URR Reporting trigger with a value “IGMP notify” (as defined in clause 4.6.X), the UPF issues an UPF report to the SMF and the corresponding IP Multicast addressing information

2c.	if the PCF had set the corresponding Policy Control Request Trigger set to “UE join to a multicast group” trigger” (as defined in clause 9.X), the SMF issuesa SMF initiated SM Policy Association Modification (as defined in 3GPP TS 23.502 [3] clause 4.16.5) reporting to the PCF the corresponding IP Multicast addressing information.

3-4.	When the UPF receives IP multicast packets from multicast server in IPTV network, the UPF select the PDU Session(s) where to transmit the multicast packets based on the multicast group, constructed in step 2 and fulfilling the FAR and QER rules described in clause 7.7.1.1.1.

NOTE:	The interactions between STB and 5G-RG are specified in TR-124 [5] in BBF and are not shown in figure 7.7.1.1-3.

##### 7.7.1.1.4	AF request to provision Multicast Access Control List information into UDR

Figure 7.7.1.1.4: AF request to provision Multicast Access Control List information into UDR

NOTE 1:	The 5GC NFs used in this scenario are assumed to all belong to the same PLMN (HPLMN).

1.	To create a new request, the AF invokes an Nnef_IPTV_configuration service operation. The request contains the Multicast Access Control List, a GPSI or an External Group Id, AF Transaction Id and may contain a DNN and/or a S-NNSAI. To update or remove an existing request, the AF invokes Nnef_IPTV_configuration_Update or Nnef_IPTV_configuration_Delete service operation providing the corresponding AF Transaction Id.

2.	The AF sends its request to the NEF. The NEF ensures the necessary authorization control, including throttling of AF requests and, as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_IPTV_configuration_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset = IPTV_configuration, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or SUPI/Internal-Group-Id).

(in the case of Nnef_IPTV_configuration_Delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset = IPTV_configuration, Data Key = AF Transaction Internal ID).

The NEF responds to the AF.

4.	The PCF(s) that have subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = IPTV_configuration, Data Key = SUPI/Internal-Group-Id) receive a Nudr_DM_Notify notification of data change from the UDR.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in TS 23.502 [3] clause 4.16.5.

Table 7.7.1.1.4-1 shows an example of a Multicast Access Control list provided by the AF in the IPTV domain to the NEF. The Multicast Access Control List defines the access right status (i.e. fully allowed, preview allowed, not allowed) of each of the Multicast channels per subscriber identified by a GPSI.

Table 7.7.1.1.4-1: Example of a Multicast Access Control list provided by the AF in the IPTV domain

|  | IP Multicast Addressing information 1 (related to Channel 1) | IP Multicast Addressing information 2 (related to Channel 2) | IP Multicast Addressing information 3 (related to Channel 3) |
| --- | --- | --- | --- |
| GPSI 1 | Fully allowed | Not allowed | Preview allowed |

The NEF maps the GPSI into the SUPI, assigned to a 5G-RG, as described in step 2 in Figure 7.7.1.1.4-1. and stores the Multicast Access Control List in the UDR as shown in Table 7.7.1.1.4-2.

Table 7.7.1.1.4-2: Example of a Multicast Access Control list stored in UDR within the Application Data Set

| DataKey | IP Multicast Addressing information 1 (related to TV Channel 1) | IP Multicast Addressing information 2 (related to TV Channel 2) | IP Multicast Addressing information 3 (related to TV Channel 3) |
| --- | --- | --- | --- |
| SUPI for 5G-RG 1 | Fully allowed | Not allowed | Preview allowed |
| SUPI for 5G-RG 2 | Fully allowed | Fully allowed | Not allowed |
| SUPI for 5G-RG 3 | Fully allowed | Preview allowed | Preview allowed |

In the case source Specific Multicast is to be used for a TV Channel, IP Multicast Addressing information corresponds to IP Multicast address and Source IP address.

The PCF is assumed to have subscribed to relevant modifications of that UDR data defined in the Table 7.7.1.1.4-2.

NEXT CHANGE

## 9.3	Policy and charging control rule

Policy and charging control rule for 5G-RG PDU Session is described in TS 23.503 [4] clause 6.3 with the clarification and difference shown in this clause.

### 9.3.1	PCC rule information to support IPTV service

-	PCF shall take Multicast Access Control list described in clause 7.7.1.1.4as input to policy decision in case of PDU Session used for IPTV service. PCC rules sent to SMF may indicate allowed IP Multicast Addressing information as defined in Table 9.3.1-1.

-	The "Gate status" is not applicable to IGMP message transmitted over PDU Session used for IPTV service.

Comparing to Table 6.3.1 in TS 23.503 [4], additional PCC rule information for PDU Session used for IPTV service is described in Table 9.3.1-1.

Table 9.3.1-1: The additional PCC rule information for PDU Session used for IPTV service

| Information name | Description | Category | PCF permitted to modify for a dynamic PCC rule in the SMF | Differences compared with table 6.3. in TS 23.203 [31] |
| --- | --- | --- | --- | --- |
| IPTV | This part defines the additional PCC rule information for PDU Session used for IPTV service. |  |  |  |
| IP Multicast traffic control information | indicates whether the service data flow, corresponding to the service data flow template, may be allowed or not allowed. (NOTE 1) | Optional | Yes | Added |
| NOTE 1: The corresponding IP Multicast Addressing information in provided in the SDF template. The SDF template may refer to “any” IP Multicast address (for example allowing the user to access to receive any IPTV channel) |  |  |  |  |

NEXT CHANGE

### 9.X	new PCRT (Policy Control Request Trigger)

The Policy Control Request Triggers relevant for SMF are described in TS 23.503 [4] clause 6.1.3.5 with the clarification and difference shown in this clause.

| Policy Control Request Trigger | Description | Difference compared with table 6.2 and table A.4.3-2 in TS 23.203 [4] | Conditions for reporting | Motivation |
| --- | --- | --- | --- | --- |
| 5G-RG join to a multicast group | The 5G-RG has joined to an IP Multicast Group. (Note 1) | new | PCF | To support IPTV as defined in clause 7.7.1 |
| 5G-RG Leave to a multicast group | The 5G-RG has left an IP Multicast Group. (Note 1) | new | PCF | To support IPTV as defined in clause 7.7.1 |
| NOTE 1: When the SMF reports this condition it indicates the corresponding IP multicast Addressing information |  |  |  |  |

NEXT CHANGE

### 4.6.3	Packet Detection Rule

PDR used to support PDU Sessions for RG follow the specificationsin TS 23.501 [2] clause 5.8.2.11.3 with the clarifications and additions shown below.

For PDU Sessions used for IPTV service,

-	Packets Filter Set support Packet Filters for IGMPmessage, including both IGMPv2 specified in RFC 2236 [x] and IGMPv3 specified in RFC 4604 [21]message.

NOTE: 	the PDI does not match on the destination/source address of the IGMP message itself, but on the (group, source) inside IGMP report message

NOTE 1: “IGMP Join” refers to the IGMP signalling issued by a host to indicate its willingness to receive traffic from an IP multicast group e.g. a Membership Report as defined in IETF RFC 2236: "Internet Group Management Protocol, Version 2 [xx] or IETF RFC 3376 [28]: "Internet Group Management Protocol, Version 3".

NOTE 2: “IGMP leave” refers to the IGMP signalling issued by a host to indicate its willingness to no more receive traffic from an IP multicast group e.g. a Leave Group as defined in IETF RFC 2236: "Internet Group Management Protocol, Version 2 [xx] or to the decision of the UPF to remove the PDU Session from the multicast replication due to the absence of renewal of the join by the RG.

### 4.6.4	Forwarding Action Rule

FAR used to support PDU Sessions for RG follow the specificationsin TS 23.501 [2] clause 5.8.2.11.6 with the clarifications and additions shown below.

For PDU Sessions used for IPTV service,

-	Following additional "Action" values are used to support IPTV service:

-	“IP Multicast Router Accept” indicate whether in case of IGMP Join to accept the multicast join and add the PDU Session to the requested multicast group distribution. This may also imply acting as a IP Multicast Router as described in clause 7.7.1.1 In that case, when UPF detect the IGMP leave message via the PDU Session, the UPF needs also to to ensure that the PDU Session is removed from the requested multicast group distribution.

-	“IP Multicast Router Reject” indicates the IP Multicast Router shall not accept  the IGMP join message.

-

### 4.6.X	Usage Reporting Rule

URR used to support PDU Sessions for RG follow the specifications in TS 23.501 [2] clause 5.8.2.11.5 with the clarifications and additions shown below:

For PDU Sessions used for IPTV service, an URR may indicate a Reporting trigger (defined in TS 23.501 [2] clause 5.8.2.11.5) with a value “IGMP notify” where the UPF is to report to the SMF when

- it adds a PDU session to the DL replication tree associated with an IP Multicast flow.

- it removes a PDU session from the DL replication tree associated with an IP Multicast flow.

The corresponding notification shall contain the (Source IP address of the DL multicast flow, Destination IP address of the DL multicast flow)

### 4.6.Y	Usage of N4 to support IPTV

Marco comment’s … MOVE the paragraph below under the clauses describing the PDR and FAR respectively and keep here. The text is revised  where needed.LTH : this clause is likely to be deleted

NOTE: The PDR and FAR described in this clause can  be implemented by a local IP multicatst Router functionality

The PDR refer to IP Multicast Addressing information related with IPTV services. In the case Source Specific Multicast is configured to be used on the PDU Session, both IP Multicast address and Source IP address shall be used to identify the TV services allowed on the PDU Session

The SMF sends sets of N4 rules for IP Multicast traffic related to IPTV service  correspond to:

- For UL in order to handle IGMP messages from 5G-RG that correspond to :

-	a PDR identifying IGMP Join messages ,

-	a FAR with an “forward to IP Multicast Router ” action in order to accept the UE requests to join an allowed multicast group. This FAR refers to forwarding the IGMP to local IP Multicast Router capabilities.

-	possibly a URR with a Reporting Trigger set to “IGMP reporting”

- For DL IP Multicast traffic:

-	a PDR identifying the IP Multicast Addressing information

-	a FAR asking to add outer header = GTP-u tunnel related with the PDU Session

-	a QER indicating the QoS to use towards the RG

Marco’s comments: we need to say that all set if send at PDU session establishment or leave open to clarify to next meeting. Prefer to leave open.

NEXT CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[6]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[7]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[8]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[9]	BBF WT-456: "AGF Functional Requirements".

[10]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The references to BBF WT-456 and WT-457 will be revised when finalized by BBF.

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[12]	BBF TR-177 Issue 1 Corrigendum 1: "IPv6 in the context of TR-101".

[13]	IETF RFC 6788: "The Line-Identification Option".

[14]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[15]	IETF RFC 3315: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[16]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[17]	IETF RFC 3633: "IPv6 Prefix Options for Dynamic Host Configuration Protocol (DHCP) version 6".

[18]	BBF TR-069: "CPE WAN Management Protocol".

[19]	BBF TR-369: "User Services Platform (USP)".

[20]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[21]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[22]	3GPP TR 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 38.413: "NG RAN; NG Application Protocol (NGAP)".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[25]	3GPP TS 22.011: "Service accessibility".

[26]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[28]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[29]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[30]	BBF TR-198: "DQS:DQM systems functional architecture and requirements".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[xx]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

END OF CHANGES