3GPP TSG-SA2 Meeting #134	S2-1908149

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1129 | rev | 7 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF Delegated Discovery and Selection for HR scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Oracle Corporation, ZTE, Deutsche Telekom AG, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-06-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A solution for delegated discovery in HR roaming and ETSUN is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide a solution for delegated SMF and PCF discovery in HR roaming and ETSUN. V-SMF discovery (6.3.2): 1. AMF sends Nsmf_PDUSession_CreateSMContext Request to SCP. The request indicates target PLMN ID for the H-SMF (HPLMN ID). The request indicates “UE location” and DNN” as discovery and selection parameters to SCP. 2. SCP selects the V-SMF from the serving PLMN. 3. V-SMF determines the need for HR roaming (based on the target HPLMN ID for the H-SMF). 4. V-SMF sends Nsmf_PDUSession_Create Request to SCP. 5. SCP discovers the H-SMF from the vNRF and hNRF based on the HPLMN ID. I-SMF discovery (5.34.3): Similar procedure is used for selecting the I-SMF as above for V-SMF selection with following differences: 1. As a first priority the AMF attempts to get the SMF discovered that supports the UE location and DNN. If the SCP discovery cannot find an SMF that supports the UE location and DNN, the SCP returns an error to AMF and AMF attempts to get the SMF discovered using only the UE location as input (i.e. I-SMF). 2. If the selected SMF determines it cannot serve the DNN, the SMF acts as an I-SMF. I-SMF sends Nsmf_PDUSession_Create Request to SCP. The request does not contain the UE location as a selection parameter. 3. Based on the lack of UE location as a selection parameter, the SCP discovers the SMF supporting the DNN. SMF Service Area: The AMF needs to be aware of the Service Area of the discovered SMF. The proposed solution is that the AMF obtains this information from the NRF after the response for PDU session creation. V-PCF discovery (6.3.7.1): Similar procedure is used for selecting the V-PCF as above for V-SMF selection: 1. AMF sends AM policy association establishment request to SCP. 2. SCP selects the V-PCF. 3. V-PCF includes the V-PCF ID in a response to AMF. 4. AMF sends UE policy association establishment request to SCP, indicating the V-PCF ID as a target. The request indicates target PLMN ID for the H-PCF (HPLMN ID). 5. SCP selects the V-PCF as indicated by the V-PCF ID. 6. V-PCF sends UE policy association establishment request to SCP, indicating the PLMN ID of the H-PCF as a discovery and selection parameter SCP. 7. SCP discovers the H-PCF based on the HPLMN ID. The related open Editor’s Notes are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs in TS 23.501 remains and the the specification will be missing procedures to handle V-SMF and I-SMF and V-PCF delegated discovery and selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.3, 6.3.2, 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The CR contains part of the same changes as CR1453 which describes how the SCP selects the PCF for AM and UE policy association in non-roaming case. Only the changes related to delegated V-PCF discovery are part of this CR. |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.34.3	I-SMF selection

The AMF is responsible of detecting when to add or to remove an I-SMF for a PDU Session. For this purpose the AMF gets from NRF information about the Service Area of SMF(s). During mobility events such as Hand-Over or AMF change, if the service area of the SMF does not include the new UE location, then the AMF inserts an I-SMF which can serve the UE location and the S-NSSAI. Conversely if the AMF detects that an I-SMF is no more needed it removes the I-SMF and interfaces directly with the SMF of the PDU Session. If the existing I-SMF cannot serve the UE location, then the AMF initiates an I-SMF change.

At PDU Session Establishment in non-roaming and roaming with LBO scenarios, if the selected SMF does not support the current UE location for the selected DNN, S-NSSAI etc, the AMF or SCP on behalf of the AMF selects an SMF for the selected DNN, S-NSSAI and in addition selects an I-SMF serving the UE location. During the lifetime of a PDU Session, the AMF initiates an I-SMF change if it determines that neither the current SMF nor the current I-SMF (if any) can serve the UE location.

When Delegated Discovery is used, the SCP performs I-SMF discovery and selection based on the discovery and selection parameters provided by AMF. When PDU Session in non-roaming and roaming with LBO scenarios is established:

-	The AMF shall send Nsmf_PDUSessionCreateSMContext Request to SCP and include the parameters as defined in clause 6.3.2 (e.g. the DNN, UE location) as discovery and selection parameters. If the SCP successfully selects an SMF matching all discovery and selection parameters, the SCP forwards the Nsmf_PDUSessionCreateSMContext Request to the selected SMF.

-	If the SCP cannot select an SMF matching all discovery and selection parameters, the SCP returns an error to AMF. Upon reception of the error, if the AMF is configured to perform discovery and selection of SMF:

-	the AMF discovers the SMF from NRF as described in this clause and includes the SMF address to the Nsmf_PDUSessionCreateSMContext Request.

The AMF shall send Nsmf_PDUSessionCreateSMContext Request to SCP and include the parameters as defined in clause 6.3.2 except the DNN as discovery and selection parameters. The SCP shall then forward the request to the I-SMF.

-	If the I-SMF decides it cannot serve the requested S-NSSAI and DNN then it decides to initiate a selection of an SMF. The I-SMF sends the Nsmf_PDUSession_Create Request towards the SMF via the SCP. If the Nsmf_PDUSessionCreateSMContext Request received from the AMF included an SMF address, the I-SMF indicates the SMF address to the SCP. If the request did not include the SMF address, the I-SMF shall include the parameters as defned in clause 6.3.2 except UE location as discovery and selection parameters to SCP.  The SCP shall then perform discovery and selection of the SMF.

-	Upon reception of a response from I-SMF, the AMF may determine the SMF Service Area of the I-SMF possibly by looking up the NRF. The AMF uses the SMF Service Area of the I-SMF to determine the need for I-SMF relocation upon subsequent UE mobility.

Comparing to the SMF selection function defined in clause 6.3.2, following parameters are not applicable for I-SMF selection:

-	Data Network Name (DNN).

-	Subscription information from UDM.

NOTE:	All SMF(s) are assumed to be able to control the UPF mapping between EPC bearers and 5GC QoS flows.

* * * * 2nd Change * * * *

### 6.3.2	SMF discovery and selection

The SMF selection functionality is supported by the AMF and SCP and is used to allocate an SMF that shall manage the PDU Session. The SMF selection procedures are described in clause 4.3.2.2.3 of TS 23.502 [3].

The SMF discovery and selection functionality follows the principles stated in clause 6.3.1.

If the AMF does discovery, the AMF shall utilize the NRF to discover SMF instance(s) unless SMF information is available by other means, e.g. locally configured on AMF. The AMF provides UE location information to the NRF when trying to discover SMF instance(s). The NRF provides NF profile(s) of SMF instance(s) to the AMF. In addition, the NRF also provides the SMF service area of SMF instance(s) to the AMF. The SMF selection functionality in the AMF selects an SMF instance and an SMF service instance based on the available SMF instances obtained from NRF or on the configured SMF information in the AMF.

NOTE 1:	Protocol aspects of the access to NRF are specified in TS 29.510 [58].

The SMF selection functionality is applicable to both 3GPP access and non-3GPP access.

The SMF selection for Emergency services is described in clause 5.16.4.5.

The following factors may be considered during the SMF selection:

a)	Selected Data Network Name (DNN).

b)	S-NSSAI.

c)	NSI-ID.

NOTE 2:	The use of NSI -ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI.

d)	Access technology being used by the UE.

e)	Support for Control Plane CIoT 5GS Optimisation.

f)	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed.

-	per S-NSSAI: the subscribed DNN(s).

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed.

-	per (S-NSSAI, subscribed DNN): whether EPC interworking is supported.

g)	Void.

h)	Local operator policies.

NOTE 3:	These policies can take into account whether the SMF to be selected is an I-SMF or a V-SMF or a SMF.

i)	Load conditions of the candidate SMFs.

j)	Analytics (i.e. statistics or predictions) for candidate SMFs' load as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

k)	UE location (i.e. TA).

l)	Service Area of the candidate SMFs.

m)	Capability of the SMF to support a MA PDU Session.

In the case of delegated discovery, the AMF, shall send all the available factors a)-d) and k) to the SCP and shall also indicate if interworking with EPS is required and if the PDU session shall be home routed. The selected SMF instance includes binding indication, including the SMF ID and SMF Set ID in the response to the AMF, as described in clause 6.3.1.0.

In addition, the AMF may indicate to the SCP which NRF to use (in case of NRF dedicated to the target slice).

Editor's note:	It is FFS how to capture that required discovery and selection information should be conveyed to the SCP with as minimal impact on the end-to-end signalling performance latency between AMF and SMF.

If there is an existing PDU Session and the UE requests to establish another PDU Session to the same DNN and S-NSSAI and the UE subscription data indicates the support for interworking with EPS for this DNN and S-NSSAI, the same SMF shall be selected. In any such a case where the AMF can determine which SMF should be selected, if delegated discovery is used, the AMF shall indicate a desired NF Instance ID so that the SCP is able to route the message to the relevant SMF. Otherwise, if UE subscription data does not indicate the support for interworking with EPS for this DNN and S-NSSAI, a different SMF may be selected. For example, to support a SMF load balancing or to support a graceful SMF shutdown (e.g., a SMF starts to no more take new PDU Sessions).

In the home-routed roaming case, the SMF selection functionality selects an SMF in VPLMN as well as an SMF in HPLMN. In the context of Network Slicing this is specified in clause 4.3.2.2.3.3 of TS 23.502 [3].

When the UE requests to establish a PDU Session to a DNN and an S-NSSAI, if the UE MM Core Network Capability indicates the UE supports EPC NAS and optionally, if the UE subscription indicates the support for interworking with EPS for this DNN and S-NSSAI, the selection functionality (in AMF or SCP) selects a combined SMF+PGW-C. Otherwise, a standalone SMF may be selected.

If the UDM provides a subscription context that allows for handling the PDU Session in the visited PLMN (i.e. using LBO) for this DNN and S-NSSAI and, optionally, the AMF is configured to know that the visited VPLMN has a suitable roaming agreement with the HPLMN of the UE, the following applies:

-	If the AMF does discovery, the SMF selection functionality in AMF selects an SMF from the visited PLMN.

-	If delegated discovery is used, the AMF indicate to the SCP that home routing is not needed, the delegated discovery and selection functionality in SCP shall discover and select an SMF from the visited PLMN.

If an SMF in VPLMN cannot be derived for the DNN and S-NSSAI, or if the subscription does not allow for handling the PDU Session in visited PLMN using LBO, then the following applies:

-	If the AMF does discovery, both an SMF in VPLMN and an SMF in HPLMN are selected, and the DNN and S-NSSAI is used to derive an SMF identifier from the HPLMN.

-	If delegated discovery is used, the AMF may be configured to perform discovery and selection of H-SMF even if delegated discovery and selection is used for discovery of other NF services. In this case the AMF discovers the H-SMF from NRF as described in this clause, and includes the target H-SMF address to the Nsmf_PDUSession_CreateSMContext Request it sends to the SCP. If the AMF is not configured to perform discovery and selection of H-SMF, the AMF shall include the HPLMN ID as a target for H-SMF in the Nsmf_PDUSession_CreateSMContext Request it sends to the SCP.

The AMF includes the DNN and UE location, the serving PLMN ID, along with other possible selection factors as discovery and selection parameters. Upon receipt of the request from the AMF, the SCP shall perform discovery and selection of the SMF in the serving PLMN. The SCP shall then forward the request to the V-SMF. If the request from AMF included the target H-SMF address, the V-SMF shall indicate the H-SMF address to SCP in the Nsmf_PDUSession_Create Request in the discovery and selection parameters it sends towards the HPLMN. If the request from AMF included the HPLMN ID as a target for H-SMF, the V-SMF shall indicate the HPLMN ID  along with other possible selection factors as discovery and selection parameters to SCP and the SCP shall perform discovery and selection of the H-SMF via the V-NRF. In this case, the SCP shall provide the HPLMN ID to the V-NRF.

-	If delegated discovery is used, upon reception of a response from I-SMF, the AMF may determine the SMF Service Area of the V-SMF possibly by looking up the NRF. The AMF uses the Service Area to determine the need for V-SMF relocation upon subsequent UE mobility.

If the initially selected SMF in VPLMN (for roaming with LBO) detects it does not understand information in the UE request, it may reject the N11 message (related with a PDU Session Establishment Request message) with a proper N11 cause triggering the AMF to select both a new SMF in the VPLMN and a SMF in the HPLMN (for home routed roaming).

The AMF selects SMF(s) considering support for CIoT 5GS optimisations (e.g. Control Plane CIoT 5GS Optimisation).

Additional details of AMF selection of an I-SMF are described in the clause 5.34.

In the case of home routed scenario, the AMF selects a new V-SMF if it determines that the current V-SMF cannot serve the UE location. The selection/relocation is same as an I-SMF selection/relocation as described in the clause 5.34.

* * * * 3rd Change * * * *

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is implemented in AMF, SMF and SCP, and follows the principles in clause 6.3.1. The AMF uses the PCF services for a UE and the SMF uses the PCF services for a PDU Session.

When NF service consumer performs discovery and selection, the following applies:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies.

-	PCF Group ID of the UE's SUPI.

NOTE:	The AMF can infer the PCF Group ID the UE's SUPI belongs to, based on the results of PCF discovery procedures with NRF. The AMF provides the PCF Group ID the SUPI belongs to other PCF NF consumers as described in TS 23.502 [3].

In the non roaming case, the AMF selects a PCF instance for AM policy association and selects the same PCF instance for UE policy association. In the roaming case, the AMF selects a V-PCF instance for AM policy association and selects the same V-PCF instance for UE policy association.

The following factors may be considered at PCF discovery and selection for Access and Mobility policies and UE policies:

-	SUPI; the AMF selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

-	S-NSSAI(s).

-	The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies.

The following factors may be considered at PCF discovery and selection for a PDU session:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	S-NSSAI of the PDU Session.

d)	SUPI; the SMF selects a PCF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for PCF discovery.

e)	PCF selected by the AMF for the UE.

f)	MA PDU Session capability of the PCF, for an MA PDU session.

g)	The PCF Group ID provided by the AMF to the SMF.

In the case of delegated discovery and selection in SCP, the SMF shall include the available factors in the first request.

Editor's note:	It still needs to be clarified which actual set of parameters the sentence above is referring to.

The selected PCF instance for serving the UE and the selected PCF instance for serving a PDU session of this UE may be the same or may be different.

In the following scenarios, information about the PCF instance that has been selected i.e. the PCF ID may be forwarded to another NF. If the NF service consumer performs discovery and selection, this NF may use this PCF instance. In the case of delegated discovery and selection, this NF may include PCF ID in the request and the SCP may use this information to select the associated PCF instance (discovery may still be needed depending on what level of information is sent by the AMF, e.g. the address of the PCF instance may not be present):

When NF service consumer performs discovery and selection, the following applies:

-	During AMF relocation, the target AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF by the target AMF, and the target AMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF ID to SMF instance(s) during the PDU Session Establishment procedure(s) to enable the usage of the same PCF for the AMF and the SMF instance(s). The SMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the H-PCF ID of the selected H-PCF instance to the V-PCF during the policy association establishment procedure.

When the SMF receives a a redirection indication with PCF ID from the PCF for the PDU session, the SMF shall terminate the current SM Policy Control association and reselects a PCF based on the received PCF ID. The SMF shall then establish an SM Policy Control association with the reselected PCF.

In the case of delegated discovery and selection in the SCP, the following applies:

-	The SCP discovers and selects a (V-)PCF instance for AM policy association. The selected (V-)PCF instance includes binding indication, including the (V-)PCF ID and PCF Group ID in the response to the AMF, as described in clause 6.3.1.0. The AMF includes the received (V-)PCF ID and PCF Group ID to the request to establish the UE policy association. The SCP selects the corresponding (V-)PCF instance for UE policy association.

-	During AMF relocation, the AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF instance by the AMF. The AMF includes that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	The AMF may, based on operator policies, forward the selected PCF ID to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF instance for the AMF and the SMF. The SMF may include that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	In the roaming case,  the AMF may be configured to to perform discovery and selection of H-PCF even if delegated discovery and selection is used for discovery of other NF services. In this case the AMF discovers the H-PCF from NRF as described in this clause, and provides the H-PCF ID in the UE policy association establishment request it sends to the SCP.

If the AMF is not configured to perform discovery and selection of H-PCF the AMF shall provide the HPLMN ID in the UE policy association establishment request it sends to the SCP. The V-PCF shall send an UE policy association establishment request towards the HPLMN. If the request from AMF included the PCF ID, the V-PCF shall indicate the H-PCF ID to SCP. Otherwise the V-PCF shall include the HPLMN ID as a discovery and selection parameter to SCP. The SCP shall then perform discovery and selection of the H-PCF via the V-NRF. The selected H-PCF instance includes the H-PCF ID and PCF Group ID in the response to the AMF.

* * * * End of Changes * * * *