3GPP TSG-SA2 Meeting #134	S2-1908610

# Sapporo, Japan, 24th June 2019 - 28th June 2019	(e-mail revision 4 of S2-1908455)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0015 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Analytics information provided by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 7.1-2 in TS 23.288 is not completed, now it only includes the analytics on slice load level information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.1-2 is filled up with other analytics, which have been proposed in TS 23.288 in clause 6. The related Editor Note below the table is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS 23.288 is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF |

The following table shows the analytics information provided by NWDAF service:

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |  |
| --- | --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Requested Analytics data, including load level information of Network Slice instance(s). |  |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience may be provided, individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice. |  |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |  |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |  |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. |  |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |  |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |  |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | The exception with an appropriate exception ID. |  |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |  |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |  |

**** End of Changes ****