3GPP TSG-SA WG2 Meeting #135	S2-1908816

# Split (Croatia), 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0004 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the PC5 unicast link modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.2.1.4 of TS 23.287 specifies that: For every PC5 unicast link, a UE self-assigns a distinct PC5 Link Identifier that uniquely identifies the PC5 unicast link in the UE for the lifetime of the PC5 unicast link. Each PC5 unicast link is associated with a Unicast Link Profile which includes: - service type(s) (e.g. PSID or ITS-AID), Application Layer ID and Layer-2 ID of UE A; and - Application Layer ID and Layer-2 ID of UE B; and - network layer protocol used on the PC5 unicast link; and - for each V2X service, a set of PC5 QoS Flow Identifier(s) (PFI(s)). Each PFI is associated with QoS parameters (i.e. PQI and optionally Range). Based on the last bullet description, it can derived that each V2X service has its own a set of PC5 QoS Flow Identifier(s) in the Unicast Link Profile which means there is a relationship between the V2X service and its own PFIs. Therefore, when UE adds new PC5 QoS Flows in an existing PC5 unicast link, the V2X Service Info(e.g. PSID(s) or ITS-AID(s)) also needs to be provided to the peer UE. if not, the peer UE can not distinguish which V2X service the new QoS flow belongs to, then the peer UE further can not derive the accept QoS info in the accept message. Since the PFI is unique within a same destination, so the PFIs are enough in the Link Modification Request message when the UE removes any existing PC5 QoS Flow(s) in an existing PC5 unicast link. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Split the original descriptions of the PC5 QoS flow handling in the existing PC5 unicast link into the following three parts : - add any new PC5 QoS Flow(s) in the existing PC5 unicast link. - modify any existing PC5 QoS Flow(s) in the existing PC5 unicast link. - remove any existing PC5 QoS Flow(s) in the existing PC5 unicast link. - Clarify that when UE adds new PC5 QoS Flows in an existing PC5 unicast link, the V2X Service Info(e.g. PSID(s) or ITS-AID(s)) needs to be included in the Link Modification Request message - when the UE removes any existing PC5 QoS Flow(s) in an existing PC5 unicast link, it only includes the PFIs in the Link Modification Request message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information elements in the current Link Modification Request message are not correct when the UE adds new PC5 QoS Flows or removes any existing PC5 QoS Flow(s)in an existing PC5 unicast link |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

6.3.3.4	Layer-2 link modification for a unicast link

Figure 6.3.3.4-1 shows the layer-2 link modification procedure for a unicast link. This procedure is used to:

-	add new V2X service(s) to the existing PC5 unicast link.

-	remove any V2X service(s) from the existing PC5 unicast link.

-	add new PC5 QoS Flow(s) in the existing PC5 unicast link.

-	modify any existing PC5 QoS Flow(s) in the existing PC5 unicast link.

-	remove any existing PC5 QoS Flow(s) in the existing PC5 unicast link.

Figure 6.3.3.4-1: Layer-2 link modification procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.3.3.1.

1.	The V2X application layer in UE-1 provides application information for PC5 unicast communication. The application information includes the service type(s) (e.g. PSID or ITS-AID) of the V2X application(s) and the initiating UE's Application Layer ID. The target UE's Application Layer ID may be included in the application information. If UE-1 decides to reuse the existing PC5 unicast link as specified in clause 5.2.1.4, so decides to modify the unicast link established with UE-2, UE-1 sends a Link Modification Request to UE-2.

The Link Modification Request message includes:

a)	To add new V2X service(s) to the existing PC5 unicast link:

-	V2X Service Info: the information about V2X Service(s) to be added (e.g. PSID(s) or ITS-AID(s)).

-	QoS Info: the information about PC5 QoS Flow(s) for each V2X Service to be added. For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

b)	To remove any V2X service(s) from the the existing PC5 unicast link:

-	V2X Service Info: the information about V2X Service(s) to be removed (e.g. PSID(s) or ITS-AID(s)).

c)	To add any PC5 QoS Flow(s) in the existing PC5 unicast link:

-	V2X Service Info: the information about V2X Service(s) to be modified (e.g. PSID(s) or ITS-AID(s)).

-	QoS Info: the information about PC5 QoS Flow(s) to be added. For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

d)	To modify any existing PC5 QoS Flow(s) in the existing PC5 unicast link:

-	QoS Info: the information about PC5 QoS Flow(s) to be modified. For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

e)	To remove any existing PC5 QoS Flow(s) in the existing PC5 unicast link:

-	QoS Info: the PFIs to be removed.

2.	UE-2 responds with a Link Modification Accept message.

The Link Modification Accept message includes:

-	For case a), case c) and case d) described in step 1:

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

The V2X layer of each UE provides information about the unicast link modification to the AS layer. This enables the AS layer to update the context related to the modified unicast link.

*** END OF CHANGES ***