3GPP TSG-SA2 Meeting #135	S2-1908893

# Split, Croatia, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0007 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on V2X communication over PC5 or Uu reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Restructure the section 5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on V2X communication over PC5 or Uu reference point in section 5.2.2 and 5.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section structure is not reasonable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First change * * * *

5.2.2	V2X communication over Uu reference point

5.2.2.1	V2X communication via unicast

The V2X communication via unicast over the Uu reference point supports non-roaming and roaming operations.

For transport of V2X messages over Uu reference point:

-	the mechanisms defined in TS 23.501 [6] and TS 23.502 [7] can be used to establish the suitable PDU Sessions, and V2X messages are routed towards V2X Application Server or towards UEs with existing unicast routing.

-	for transport of non-IP based V2X messages from the application:

-	IP encapsulation (i.e. IP PDU Session type) or Unstructured PDU Session type is used.

-	The UE determines which method between IP encapsulation and Unstructured PDU Session type is used for non-IP based V2X messages based on the UE configuration as described in clause 5.1.3.1. If no such configuration is available, UE can use the method based on the UE implementation.

-	when IP PDU Session type is used for transport of IP based or non-IP based V2X messages, V2X messages are transported;

-	regarding UDP or TCP,

-	for non-IP based V2X messages from the application, UE may learn from V2X Application Server or UE may be configured explicitly about a transport layer protocol needs to be used or no restriction imposed by configuration.

-	for IP based V2X messages from the application, UE uses the transport layer protocol set by the upper layer.

-	the UE sends a V2X message to a V2X Application Server address. The destined V2X Application Server address is derived from the PSID or the ITS-AID and the UE configuration as described in clause 5.1.3.1, and the V2X Application Server receives the V2X message in a UDP/IP packet or a TCP/IP packet on a V2X Application Server address.

-	when Unstructured PDU Session type is used for transport of non-IP based V2X messages:

-	V2X messages are transported to the V2X Application Server as defined in clause 5.6.10.3 of TS 23.501 [6].

5.2.3	V2X communication over PC5 or Uu reference point

5.2.3.1	General

V2X communication over PC5 reference point may use different protocols and formats than V2X communication over Uu reference point.

For an application (identified by PSID or ITS-AID) that can use either PC5 reference points or Uu reference point for the transmission of the same V2X messages, the additional consideration for transport of V2X messages over Uu reference point is described in section 5.2.2.1.

-	for transport of non-IP based V2X messages from the application:

-	IP encapsulation (i.e. IP PDU Session type) or Unstructured PDU Session type is used.

-	The UE determines which method between IP encapsulation and Unstructured PDU Session type is used for non-IP based V2X messages based on the UE configuration as described in clause 5.1.3.1. If no such configuration is available, UE can use the method based on the UE implementation.

-	when IP PDU Session type is used for transport of IP based or non-IP based V2X messages, V2X messages are transported;

-	regarding UDP or TCP,

-	for non-IP based V2X messages from the application, UE may learn from V2X Application Server or UE may be configured explicitly about a transport layer protocol needs to be used or no restriction imposed by configuration.

-	for IP based V2X messages from the application, UE uses the transport layer protocol set by the upper layer.

-	the UE sends a V2X message to a V2X Application Server address. The destined V2X Application Server address is derived from the PSID or the ITS-AID and the UE configuration as described in clause 5.1.3.1, and the V2X Application Server receives the V2X message in a UDP/IP packet or a TCP/IP packet on a V2X Application Server address.

-	when Unstructured PDU Session type is used for transport of non-IP based V2X messages:

-	V2X messages are transported to the V2X Application Server as defined in clause 5.6.10.3 of TS 23.501 [6].

Latency reduction for V2X message transfer via unicast may be achieved by using various mechanisms, including via "Application Function influence based edge computing for V2X services" in Annex A, where the V2X Application Server acts as an Application Function.

# * * * End of changes * * * *