3GPP TSG-WG SA2 Meeting #135	S2-1909250

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1780 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call flows for DDD status event with UPF buffering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the DDD status event subscription, the following EN in 4.15.3.2.5 need to be resolved: A procedure to subscribe to data delivery status notifications if buffering in the UPF is used is ffs. When the UP connection of the PDU Session is deactivated and the SMF decides to activate buffering in the UPF for the session, the SMF shall inform the UPF to start buffering packets for this PDU Session. When the UPF receives downlink data, it triggers the network triggered service request procedure in clause 4.2.3.3. In the procedure, when the SMF is informed the UE is unreacheable by the AMF, the SMF determines Extended Buffering does not apply or apply accordingly. To support the DDD event notification in case of buffering in UPF applies, the following scenarios are needed to be considered: - When the SMF receives the subscription from UDM, for a UE’s specific PDU session, the SMF has to be informed the UE is unreachable and extended buffer is activated. - When the SMF receives the subscription from UDM for a UE’s specific PDU session the SMF has not be informed the UE is unreachable. When DN is received by the SMF and the SMF is informed by the AMF, the SMF determines to activate the extended buffer in the UPF. For the above two cases, when the SMF receives the DDD status event subscription, the SMF can further request the UPF forward the new traffic information of buffered data from now on, and when the UPF determines to discard the buffered information, it will also forward the traffic information of the discard data. Based on the notification, the SMF can know the DDD status. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify how to subcribe to data delivery status notifications if buffering in the UPF is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If extended buffer in SMF is not supported, Data delivery status notifications can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR1780 modifies Figure 4.15.3.2.5-1 (in a non-overlapping way), so the figures will need to be merged in implementation of the CRs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2.5	Information flow for downlink data delivery status

The procedure is used if the SMF requests the UPF to forward packets that are subject of extended buffering in the SMF or configures the relevant UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to data delivery status notifications. The data delivery status notifications relates to high latency communication, see also clauses 4.24.2 and 4.2.3.3.

The NEF service operations information flow in clause 4.15.3.2.3 is applied according to the detailed flow in the present clause.

Figure 4.15.3.2.5-1: Information flow for downlink data delivery status

0.	The SMF (in the non-roaming case the H-SMF. in the roaming case the V-SMF) configures the relevant UPF to forward packets that are subject to extended buffering or configures the relevant UPF to buffer packets. The SMF decides to apply this behavior based on the "expected UE behaviour". Alternatively, step 0 is triggered by step 3,

1.	AF sends Nnef_EventExposure_Subscribe Request to NEF requesting data delivery status for a UE or group of UEs and identifying the source of the downlink IP or Ethernet traffic. The Downlink delivery status events include:

-	First downlink Packet(s) in extended buffering event:

-	This event is triggered when the first new downlink data packet is buffered with extended buffering matching the source of the downlink traffic.

-	in notifications about this Downlink delivery status, the SMF provides the Extended Buffering time as determined in clause 4.2.3.3.

-	First downlink Packet discarded:

-	This event occurs when the first packet matching the source of the downlink traffic is discraded because the Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded.

-	First Downlink Packet transmitted:

-	This event occurs when the first packet matching the source of the downlink traffic information is transmitted after previous buffering or discarding of corresponding packet(s) because the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE according to clause 4.2.3.3.

2.	NEF sends the Nudm_EventExposure_Subscribe Request to UDM. Identifier of the UE or group of UEs, the source of the downlink traffic , monitoring event received from AF in step 1, and NEF notification endpoint are included in the message.

3.	UDM sends the Nsmf_EventExposure_Subscribe Request message to each SMF where at least one UE identified in step 2 has a PDU session established and includes identifier of the UE or Internal-Group-Id, the source of the downlink traffic, monitoring event and the notification endpoint of NEF received in step 2. If the UDM becomes aware that such a UE has a PDU session established at a later time than when receiving step 2, the UDM then executes step 3.

4.	SMF sends the Nsmf_EventExposure_Subscribe Response message to UDM.

5.	UDM send sends the Nsmf_EventExposure_Subscribe response message to NEF.

6.	NEF sends the Nsmf_EventExposure_Subscribe response to AF.

7.	In case of the buffering in SMF applies, the SMF detects a change in Downlink Delivery Status event as described in clause 4.2.3. The SMF becomes aware that Downlink Packet(s) require extended buffering when trying to send packets to the AMF via a Namf_Communication_N1N2MessageTransfer service operation and obtaining a corresponding respone. If the the SMF decides to discard packets, the "Downlink Packet(s) discarded event" is detected. The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

In case of buffering in the UPF applies, when the SMF is informed that the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation. In addition to the behaviour described in step 3c of clause 4.2.3.3, the SMF requests the UPF to report the traffic information (e.g. Source IP address, Source port number) of the buffered downlink packets per source of down traffic and/or discard downlink packets per source of down traffic in the UPF. The SMF determines the DDD status by comparing the traffic information reported by the UPF with the Traffic Descriptor(s) received in the event subscription(s). The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

8.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

9.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.

# * * * * End of changes * * * *