SA WG2 Meeting #135	S2-1909260

October 14-18, Split, Croatia	(revision of )

Source:	Nokia, Nokia Shanghai Bell, Telecom Italia

Title:	KI on limitation of number of connections per Network Slice

Document for:	Discussion and Agreement

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2/Rel-17

Abstract of the contribution:

# Discussion

One of the attributes in the GST documented in NG.116 is the following:

    - Number of connections

This attribute describes the maximum number of concurrent sessions supported by the network slice.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | NA |
| Example | 100.000 sessions 10.000.000 sessions |
| Tags | Scalability attribute |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| Optional |  |

Number of Connections Table

There is no way for a PLMN to support this limitation in the PLMN and also when the UEs are roaming. In fact, currently a SMF may limit PDU sessions per DNN but not different DNNs in same slice may use different SMFs and so cross SMF limitations cannot be supported. How to support this attribute in the system needs to be studied.

# Proposal

It is proposed the following key issue is added to TS 23.700-40

Proposed Text

## 5.X	Key Issue #X: limitation of number of connections per slice

### 5.X.1	General description

One of the attributes in the GST documented in GSMA 5GJA NG.116[x] is the following:

- Number of connections This attribute describes the maximum number of concurrent sessions supported by the network slice. Parameters. Value; Integer. Measurement unit; NA. Example; 100.000 sessions 10.000.000 sessions. Tags; Scalability attribute. Attribute Presence. Mandatory. Conditional; X. Optional. Number of Connections Table

There is no way for a PLMN to support this limitation as currently a SMF may limit PDU sessions per DNN but not different DNNs in same slice, which may also use different SMFs and so cross SMF limitations cannot be supported.

- This key issue shall study how to detect that a number of PDU sessions for a certain S-NSSAI value has been exceeded.

- The solution shall enable an operator to apply the limitation only the S-NSSAIs that require this limitation based on the NEST.

- The non-roaming and roaming cases shall be supported for Home Routed and non-Home Routed (LBO) cases.

- The actions taken towards the UE that has exceeded the limit shall be studied

- Whether to enforce fairness across UEs that are using the slice may be studied, e.g.. how to avoid that a UE has N connection and another UE is unable to get service

- Any charging aspects will need to be identified in cooperation with SA5