SA WG2 Meeting #135	S2-1909292

October 14-18, Split, Croatia	(revision of )

Source:	Nokia, Nokia Shanghai Bell

Title:	KI on limitation of data rate per Network Slice in UL and DL per UE.

Document for:	Discussion and Agreement

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2, Rel-17

Abstract of the contribution:

# Discussion

One of the attributes in the GST documented in NG.116 is the following:

Maximum downlink throughput

This attribute describes the maximum data rate supported by the network slice per UE in downlink. These parameters could be used to offer different contract qualities like gold, silver and bronze.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | Bronze customer: 50.000 Kbps Silver customer: 400.000 Kbps Gold customer: 1.000.000 Kbps |
| Tags | Character attribute /Performance KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

Maximum uplink throughput per UE

This attribute describes the maximum data rate supported by the network slice per UE in uplink. These parameters could be used in order to offer different contract qualities like gold, silver and bronze.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | Bronze customer: 10.000 Kbps Silver customer: 100.000 Kbps Gold customer: 200.000 Kbps |
| Tags | Character attribute / Functional KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

There is no way for a PLMN to support this limitation for a UE as so far it is possible to perform per PDU session or per UE data rate limitation, but not per network slice/UE.

This KI addresses this problem space.

# Proposal

It is proposed the following key issue is added to TS 23.700-40

Proposed Text

## 5.X	Key Issue #X: limitation of data rate per network slice in UL and DL per UE.

### 5.X.1	General description

One of the attributes in the GST documented in GSMA 5GJA NG.116[x] is the following:

Maximum downlink throughput This attribute describes the maximum data rate supported by the network slice per UE in downlink. These parameters could be used to offer different contract qualities like gold, silver and bronze. Parameters. Value; Integer. Measurement unit; kbps. Example; Bronze customer: 50.000 Kbps Silver customer: 400.000 Kbps Gold customer: 1.000.000 Kbps. Tags; Character attribute /Performance KPI. Attribute Presence. Mandatory. Conditional. Optional; X. Maximum uplink throughput per UE This attribute describes the maximum data rate supported by the network slice per UE in uplink. These parameters could be used in order to offer different contract qualities like gold, silver and bronze. Parameters. Value; Integer. Measurement unit; kbps. Example; Bronze customer: 10.000 Kbps Silver customer: 100.000 Kbps Gold customer: 200.000 Kbps. Tags; Character attribute / Functional KPI. Attribute Presence. Mandatory. Conditional. Optional; X.

There is no way for a PLMN to support per slice data rate limitation for a UE as so far it is possible to perform per PDU session (session-AMBR) or per UE data rate limitation (UE-AMBR, see TS 23.501[x]), but not per network slice/UE.

This key issue shall study:

- how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL.

- Interaction with the rates guaranteed for GBR and delay critical GBR QoS flows in the aggregate of PDU sessions

- Signalling and subscription data aspects shall be considered.

- Any RAN impacts shall be identified and alignment with RAN WGs shall be pursued if any impacts are identified.