SA WG2 Meeting #S2-135	S2-1909401

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1804 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN on Traffic Forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In this CR paper we address the EN on Traffic Forwarding information: “Traffic forwarding information being not available in IEEE 802.1Qbv and how it is received is FFS.” In Rel. 16, we propose to adopt general community recommendation that the CNC configures the forwarding rules statically. This information is general for any bridge, and not specific for TSN (not from IEEE 802.1Qbv). CNC delivers the forwarding traffic information related to the path of every TSN Stream to the TSN AF, but this information is not linked to a TSN Stream ID, but to the destination MAC address, Traffic class, VLAN ID, and involved ports. Dynamically built forwarding tables that define the traffic forwarding rules is a more complex process that can be handled if needed in a future release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the editor note on Traffic Forwarding information. Detach Traffic Forwarding information as part of IEEE 802.1Qbv. Indicate that the forwarding rules may be configured statically by the CNC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved issue pointed out by this EN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
| different traffic classes can not be mapped into same QoS flowed. |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.28.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session.

Editor's note:	For the UL TSN traffic, how to correlate the egress port in the configuration to PDU session is FFS.

The configuration information of 5GS Bridge as defined in 802.1Qbv [96], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities;

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

Traffic forwarding information is configured statically by CNC and includes:

-		Destination MAC address, Traffic class and VLAN ID of TSN stream;

-	The ingress port number and egress port number on 5GS Bridge.

*** END CHANGES ***