SA WG2 Meeting #S2-135	S2-1909404

14 - 18 October, 2019, Split Croatia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1807 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for UL configuration issue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Remove EN for UL TSN traffic |  |  |  |  |  |  |  |  |
| Summary of change: |  | and per-stream filtering and policing added for TTs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Consider changing Qbv to IEEE 802.1Q which includes Qbv & Qci. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.28.2	QoS parameters mapping for 5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session.

Editor's note:	Traffic forwarding information being not available in IEEE 802.1Qbv and how it is received is FFS.

The configuration information of 5GS Bridge as defined in 802.1Qbv [96], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic and per-stream filtering and policing as defined in IEEE 802.1Qci [xx] on ports of DS-TT and NW-TT:

-	Egress and ingress ports of 5GS Bridge;

-	Traffic classes and their priorities;

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Qbv [96], Annex Q.2 for 5GS.

-	Traffic forwarding information:

-	Destination MAC address, Traffic class and VLAN ID of TSN stream;

-	The ingress port number and egress port number on 5GS Bridge.

The association between the MAC address used by the PDU Session, 5GS Bridge ID and port number on DS-TT is maintained and further used to assist to bind the TSN traffic with the UE.

In the case of provisioning traffic forwarding information, the AF determines the MAC address used by the PDU Session for the TSN traffic, and requests to influence traffic routing as defined in clause 5.6.7 with the DS-TT MAC address of the PDU Session, traffic class, stream destination MAC address, ingress port number or egress port number on the NW-TT side and VLAN ID.

*** END CHANGES ***