3GPP TSG-SA2 Meeting #135	S2-1909542

# October 14 – 18, 2019, Split, HR

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1819 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the binding procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In clause 4.17.12.2, the SCP handling of the binding indication is not clear in step 3. - In clause 4.17.12.3, the SCP handling of the binding indication is not clear in step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.17.12.2, adding the SCP handling of the binding indication in step 3. - In clause 4.17.12.3, if binding is created as the service request, the binding indication will be included in the service request from instance A. In this case, the SCP will forward this binding indication to instance B. The SCP handling is added in step 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCP handling of the binding indication is not clear in clause 4.17.12.2 and 4.17.12.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.12.2, 4.17.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

4.17.12.2	Binding created as part of service response

When the NF service consumer communicates with the NF service producer, the producer may return a binding indication to the consumer. The consumer stores the received binding indication and uses it for the subsequential requests concerning the data context.

Figure 4.17.12.2-1: Binding created as part of service response

1.	If Direct Communication is used, the NF service consumer selects the NF service producer and sends the request to the selected NF service producer. If Indirect Communication mode is used the NF service consumer sends the request to the SCP and provides within the service request to the SCP the discovery and selection parameters necessary to discover and select a NF service producer.

2.	The NF service producer sends a response to the NF service consumer. In the response the NF service producer may include a binding indication. If the NF service consumer receives a resource information and binding indication as specified in Table 6.3.1.0-1 of TS 23.501 [2], it uses them for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

3.	The NF service consumer uses the binding indication and resource information received in the previous step for subsequent requests regarding the concerned resource. If dedicated discovery is used, the SCP shall route the service request using the binding indication sent from the NF service consumer.

4.	The NF service producer sends a response to the consumer. The NF service producer may respond with an updated resource information and binding indication, or both, different to the one received in the previous response.

**************** Second Change ****************

4.17.12.3	Binding created as part of service request

If the NF service consumer can also be as a NF service producer for later communication from the contacted producer, a service request sent to the producer may include binding indication.

Figure 4.17.12.3-1: Binding created as part of service request

1.	Instance A, as an NF service consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF service producer. If Instance A, can also be NF service producer for later communication for the concerned data context, it may include binding indication referring to NF service instance, NF instance or NF Set as specified in Table 6.3.1.0-1 of TS 23.501 [2] in the request sent to the NF service producer. In this case, if indirect communication is used, the SCP sends to the Instance B the service request including the binding indication.

2.	Instance B as the NF service producer sends a response to the NF service consumer.

3.	When Instance B as NF service consumer needs to invoke the service provided by Instance A, Instance B sends a request using the binding indication received in step 1 as described in Steps 3-4 in Figure 4.17.12.2-1 with the following difference:

-	Based on the received binding indication, if delegated discovery is not used, the Instance B may need to discover the corresponding endpoint address of the Instance A.

**************** END CHANGES **************