3GPP TSG-SA2 Meeting #135	S2-1909579

# Splt, Croatia, October 14, 2019 - October 18, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0078 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User Data Congestion - Removal of Editor's Notes and Description Alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 10-02-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The general description and input data sections of the User Data Congestion Analytics procedure describe that the NWDAF collects measurement information from the OAM system and then generates user data congestion related analytics. However, in the detailed procedure descriptions, it is stated that the NWDAF will “get the user data congestion status for the requested location”. This needs to be corrected; the NWDAF does not get congestion information from the OAM, rather, as stated in section 6.8.1 “The NWDAF then collects measurements per cell and uses the measurements to determine user data congestion analytics.” There is 1 editor’s notes that states “The exact performance measurements (or KPIs) that are collected from the OAM need to be coordinated with SA5”. There are 4 editor’s notes that state “The exact SA5 service used is FFS”. In their LS, S2-1908724, SA5 indicated that “PDCP data volume measurements, RF measurements, UE throughput measurements etc. are related to measure RAN network load” and further pointed out that reporting mechanisms in TS 28.532 and TS 28.550 can be used by the NWDAF to collect measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the editor’s note that stated “The exact performance measurements (or KPIs) that are collected from the OAM need to be coordinated with SA5”. Added a statement pointing out that that reporting mechanisms in TS 28.532 and TS 28.550 can be used by the NWDAF to collect measurements. The examples that were listed in the SA5 LS are already part of the existing text. Removed the 4 editor’s notes that stated “The exact SA5 service used is FFS” and replaced each editor’s note with a sentence that says “The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].” In 2 steps of procedure of section 6.8.4.1, changed it so that it says the NWDAF collects measurement information from OAM Services, instead of collecting congestion information. In 2 steps of procedure of section 6.8.4.2, changed it so that it says the NWDAF collects measurement information from OAM Services, instead of collecting congestion information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification would have conflicting statements and editor’s notes would remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.2, 6.8.4.1, 6.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### *********************************************Change 1***********************************************

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

Table 6.8.2-1: Service Data from the NF related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE Location | AMF | UE location information that maybe provided to the NWDAF when the NWDAF is used derive the Area of Interest. |
| AMF ID | UDM | The AMF ID to be used for obtaining the UE's location. |
| Performance Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

#### *********************************************Change 2***********************************************

#### 6.8.4.1	Procedure for one-time or continuous reporting of analytics for user data congestion in a geographic area

The procedure as depicted in Figure 6.8.4.1-1 is used by an NF to retrieve congestion analytics for a specific geographic area. The procedure can be used to request a one-time or continuous reporting of congestion analytics.

Figure 6.8.4.1-1: Procedure for one-time or continuous reporting of analytics for congestion in a geographic area

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, indicating request for analytics for congestion in a specific location. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, Analytics Filter is set to a location (e.g. ECGI, TA).

2-3. If the request is authorized, and in order to provide the requested analytics, the NWDAF may request measurement information for the requested location from OAM services following the data collection from OAM procedure as captured in 6.2.3.2. If the NWDAF already has information about the requested location, these steps are omitted. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

6.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF, indicating request for analytics for congestion in a specific location (e.g. ECGI, TA), possibly with thresholds. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

7-8. The NWDAF subscribes to OAM services following the data collection from OAM procedure as captured in 6.2.3.2 to get measurement information for the requested location, possibly providing measurement thresholds. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

9.	The NWDAF derives requested analytics.

10.	The NWDAF provides the analytics for congestion to the NF.

11.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

12.	The NWDAF derives new analytics.

13.	The NWDAF provides a notification for analytics for the user data congestion to the NF.

#### *********************************************Change 3***********************************************

#### 6.8.4.2	Procedure for one-time or continuous reporting of analytics for user data congestion for a specific UE

The procedure as depicted in Figure 6.8.4.2-1 is used by an NF to retrieve user data congestion analytics for a specific UE. The procedure can be used to request a one-time or continuous reporting of user data congestion analytics.

Figure 6.8.4.2-1: Procedure for one-time or continuous reporting of analytics for congestion for a specific UE

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, requesting for analytics for user data congestion for a specific UE id. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, the Target of Analytics Reporting is set to UE id.

2-5. The NWDAF may already know the UE location. If not, the NWDAF checks the UE location by first retrieving the AMF serving the UE (steps 2-3) and then by interrogating the AMF about the UE location.

6-7. The NWDAF requests measurement information for the UE location from OAM services (as captured in 6.2.3.2). These steps are omitted if the NWDAF already has the information. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

8.	The NWDAF derives requested analytics.

9.	The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

10.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF. The NF can request for statistics or for predictions or for both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

11.	The NWDAF determines the UE location, either via internal information or by applying the same steps as steps 2 to 5. The NWDAF then determines an area of interest.

12-13. The NWDAF subscribes to OAM services (as captured in 6.2.3.2) to get the measurement information for the UE location, possibly providing measurement thresholds. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

14.	The NWDAF derives requested analytics.

15.	The NWDAF provides the analytics for user data congestion status information to the NF.

16-17. The NWDAF subscribes to UE mobility event notification in order to be informed when the UE moves out of the area of interest (in order to define a new area of interest and request new information to OAM if the UE moves to a different area).

18.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

19.	The NWDAF derives new analytics.

20.	The NWDAF provides a notification for analytics for the user data congestion status information to the NF.