3GPP TSG-WG SA2 Meeting #135 	S2-1909680

# Split, Croatia, October 14 – 18, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0083 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the analytics exposure of the OSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Firstly, the Observed Service Experience (OSE) analytics description in clause 6.4.1 does not align with the OSE definition for UEs on a Network Slice in clause 6.4.5, TS 23.288. Secondly, during the OSE analytics subscription or request, it is unclear whether all the Analytics Filter information are mandatory or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the Observed Service Experience analytics description; - Clarify the presence (mandatory or optional or conditional) of the Analytics Filter information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The OSE analytics description in clause 6.4.1 is not clear. - The presence of the Analytics Filter information in clause 6.4.3 is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for UEs (i.e. for a UE or a group of UEs or a list of UEs or any UE) on a Network Slice, given an Application or a set of Applications or any Application (i.e. all Applications) in the Network Slice;

-	Service Experience for UEs (i.e. for any UE) in an Application.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Presence | Description |
| --- | --- | --- |
| Application ID (1..n) | Conditional (NOTE 1) | An identification of the application or a set of identifications of the applications for which the analytics information is subscribed or requested. (NOTE 2) |
| S-NSSAI | Conditional (NOTE 3) | Identifies the Network Slice for which analytics information is subscribed or requested. |
| Area of Interest | Optional | Identifies the Area where the analytics information is subscribed or requested. |
| Media/application bandwidth | Optional | Identifies the Media/application bandwidth requirement of the application. |
| DNN | Optional | DNN of the application. |
| DNAI | Optional | Location of the application. |
| NOTE 1: If no Application ID is provided, the Analytics Filter information applies any application (i.e. all applications) in the Network Slice. NOTE 2: If one Application ID is provided, the Analytics Filter information applies to the Service Experience for UEs in an application or to the Service Experience for UEs on a Network Slice subscription or request. If a set of Application IDs or any Application ID (i.e. all applications) are provided, the Analytics Filter information applies to the Service Experience for UEs on a Network Slice subscription or request. NOTE 3: The S-NSSAI is mandatory if the NWDAF Service Consumer subscribes or requests the Service Experience for UEs on a Network Slice. |  |  |

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provides statistics and predictions on the observed service experience to a consumer NF or to OAM.

NOTE 4:	OAM uses the Observed Service Experience as defined in SA WG5 TSs.

# * * * * Next change * * * *

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

Table 6.4.3-1: Service Experience statistics

| Information | Presence | Description |
| --- | --- | --- |
| S-NSSAI | Optional | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) |  | List of observed service experience information for each Application. |
| > Application ID | Optional | Identification of the application. |
| > Service Experience Type | Optional | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Mandatory | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | Optional | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Optional | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Optional | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Optional | Validity period as defined in clause 6.1.3. |
| Slice service experience | Optional | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-2: Service Experience predictions

| Information | Presence | Description |
| --- | --- | --- |
| S-NSSAI | Optional | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) |  | List of predicted service experience information for each Application. |
| > Application ID | Optional | Identification of the application. |
| > Service Experience Type | Optional | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Mandatory | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | Optional | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Optional | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Optional | Area, where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Areas of Interest. |
| > Validity period | Optional | Validity period as defined in clause 6.1.3. |
| > Confidence information | Optional | Confidence of this prediction. |
| Slice service experience | Optional | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |

The number of Service Experiences and SUPIs is limited by the maximum number of results provided as input parameter.

# * * * * End of changes * * * *