3GPP TSG-SA WG2 Meeting #135	S2-1909808

14 - 18 October, 2019, Split Croatia	(revision of S2-190xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1901 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of IAB-indication to 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the IAB operation, there is a need for the IAB-node to indicate to NG-RAN and 5GC that IAB enhancements need to be applied. In SA2#134, the mechanism to support such indication was left open, as documented in clause 5.35.2: Editor's note: Whether this indication is provided by the IAB-node at RRC level will be determined by RAN WGs. In the RAN3 LS (S2-1908697/R3-194787), it is clarified that RAN3 requires the indication at RRC level, and uses it for the CN node selection: RAN3 presently assumes that the RAN receives an IAB-indication from the IAB-node in RRC message 5 (RRC SETUP COMPLETE). Based on this indication, the RAN selects an AMF/MME supporting IAB. Additionally, this indication is to be included in the INITIAL UE MESSAGE to the 5GC, e.g. AMF, to trigger the IAB authorization. Therefore, TS 23.501 needs to be updated to reflect the above operation principles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor's note, and add the description on how the IAB-indication is sent to the 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB operation is not supported in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.2; 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.35.2	5G System enhancements to support IAB

In IAB operation, the IAB-node MT interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401 [42].

Editor's note: The following are the expected system enhancements to assist the development of the CRs for IAB. It will be revised and updated based on RAN WG's conclusion.

For the IAB-node MT operation, the existing UE authentication methods as defined in TS 33.501 [29] applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

Editor's note:	Security aspect is being studied by SA WG3, and the above will be revised and aligned after the conclusion of the study.

The following aspects are enhanced to support the IAB operation:

-	the Registration procedure as defined in TS 23.502 [3] clause 4.2.2.2 is enhanced to indicate IAB-node's capability to the AMF;

-	An IAB-indication is provided to the IAB-donor-CU when the IAB-node establishes the RRC connection as defined in TS 38.331 [28]. When this indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34]  so that the AMF can perform IAB authorization.

-	the UE Subscription data as defined in TS 23.502 [3] clause 5.2.3 is enhanced to include the authorization information for the IAB operation;

-	Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information;

-	UE Context setup/modification procedure is enhanced to provide IAB authorized indication to NG-RAN.

After registered to the 5G system, the IAB-node remains in CM-CONNECTED state. In the case of radio link failure, the IAB-node MT uses existing UE procedure to restore the connection with the network. The IAB-node MT uses Deregistration Procedure as defined in TS 23.502 [3] clause 4.2.2.3 to disconnect from the network.

*** NEXT CHANGE ***

### 6.3.5	AMF discovery and selection

The AMF discovery and selection functionality is applicable to both 3GPP access and non-3GPP access.

The AMF selection functionality can be supported by the 5G-AN (e.g. RAN, N3IWF) and is used to select an AMF instance for a given UE. An AMF supports the AMF selection functionality to select an AMF for relocation or because the initially selected AMF was not an appropriate AMF to serve the UE (e.g. due to change of Allowed NSSAI). Other CP NF(s), e.g. SMF, supports the AMF selection functionality to select an AMF from the AMF set when the original AMF serving a UE is unavailable.

5G-AN selects an AMF Set and an AMF from the AMF Set under the following circumstances:

1)	When the UE provides no 5G-S-TMSI nor the GUAMI to the 5G-AN.

2)	When the UE provides 5G-S-TMSI or GUAMI but the routing information (i.e. AMF identified based on AMF Set ID, AMF pointer) present in the 5G-S-TMSI or GUAMI is not sufficient and/or not usable (e.g. UE provides GUAMI with an AMF region ID from a different region).

3)	AMF has instructed AN that the AMF (identified by GUAMI(s)) is unavailable and no target AMF is identified and/or AN has detected that the AMF has failed.

In the case of NF Service Consumenr based discovery and selection, the CP NF selects an AMF from the AMF Set under the following circumstances:

-	When the AMF has instructed CP NF that a certain AMF identified by GUAMI(s) is unavailable and the CP NF was not notified of target AMF; and/or

-	CP NF has detected that the AMF has failed; and/or

-	When the selected AMF does not support the UE's Preferred Network Behaviour.

In the case of delegated discovery and associated selection, the SCP selects an AMF from the corresponding AMF Set under the following circumstances:

-	The SCP gets an indication "select new AMF within SET" from the CP NF; and/or

-	SCP has detected that the AMF has failed.

The AMF selection functionality in the 5G-AN may consider the following factors for selecting the AMF Set:

-	AMF Region ID and AMF Set ID derived from GUAMI.

-	Requested NSSAI.

-	Local operator policies.

-	5G CIoT features indicated in RRC signalling by the UE.

-	IAB support capability.

AMF selection functionality in the 5G-AN or CP NFs or SCP considers the following factors for selecting an AMF from AMF Set:

-	Availability of candidate AMF(s).

-	Load balancing across candidate AMF(s) (e.g. considering weight factors of candidate AMFs in the AMF Set).

-	In 5G-AN, 5G CIoT features indicated in RRC signalling by the UE.

When the UE accesses the 5G-AN with a 5G-S-TMSI or GUAMI that identifies more than one AMF (as configured during N2 setup procedure), the 5G-AN selects the AMF considering the weight factors.

When 5G-S-TMSI or GUAMI provided by the UE to the 5G-AN contains an AMF Set ID that is usable, and the AMF identified by AMF pointer that is not usable (e.g. AN detects that the AMF has failed) or the corresponding AMF indicates it is unavailable (e.g. out of operation) then the 5G-AN uses the AMF Set ID for selecting another AMF from the AMF set considering the factors above.

The discovery and selection of AMF in the CP NFs or SCP follows the principle in clause 6.3.1

In the case of NF Service Consumer based discovery and selection, the AMF or other CP NFs shall utilize the NRF to discover the AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AMF or other CP NFs. The NRF provides the NF profile(s) of AMF instance(s) to the AMF or other CP NFs. The AMF selection function in the AMF or other CP NFs selects an AMF instance as described below:

When NF Service Consumer performs discovery and selection the following applies:

-	In the case of AMF discovery and selection functionality in AMF or other CP NFs use GUAMI, TAI to discover the AMF instance(s), the NRF provides the NF profile of the associated AMF instance(s). If the associated AMF is unavailable due to AMF planned removal, the NF profile of the backup AMF used for planned removal is provided by the NRF. If the associated AMF is unavailable due to AMF failure, the NF profile of the backup AMF used for failure is provided by the NRF. If no AMF instances related to the indicated GUAMI can be found or AMF Pointer value used by more than one AMF is found, a list of NF profiles of candidate AMF instances in the same AMF Set is provided by the NRF. The NF profile may contain priority, TAI(s) and other relevant information. In this case, other CP NF can select any AMF instance from the list of candidate AMF instances.

-	In the case of AMF discovery and selection functionality in AMF use AMF Set ID to discover AMF instance(s), the NRF provides a list of NF profiles of AMF instances in the same AMF Set, together with additional information.

-	At intra-PLMN mobility, the AMF discovery and selection functionality in AMF can use AMF Set ID, AMF Region ID, the target location information, S-NSSAI(s) of Allowed NSSAI to discover target AMF instance(s). The NRF provides the target NF profiles matching the discovery.

-	At inter PLMN mobility, the source AMF selects an AMF instance(s) in the target PLMN by querying target PLMN level NRF via the source PLMN level NRF with target PLMN ID. The target PLMN level NRF returns an AMF instance address based on the target operator configuration. After the Handover procedure the AMF may select a different AMF instance as specified in clause 4.2.2.2.3 in TS 23.502 [3].

In the context of Network Slicing, the AMF selection is described in clause 5.15.5.2.1.

When delegated discovery and associated selection is used, the following applies:

-	If the CP NF includes GUAMI and TAI in the request the SCP selects will send the request to a GUAMI associated an AMF service instance that is associated with the GUAMI if it is available.

-	If none of the AMF service instances associated with the GUAMI associated AMF are available due to AMF planned removal, an AMF service instance from the backup AMF used for planned removal is selected by the SCP.

-	If none of the AMF service instances associated with the GUAMI associated AMF are available due to AMF failure, an AMF service instance from the backup AMF used for failure is selected by the SCP. If no AMF service instances related to the indicated GUAMI can be found or AMF Pointer value used by more than one AMF is found, an AMF instance in the same AMF Set is selected.

-	If the CP NF includes AMF Set ID in the request, the SCP selects AMF/AMF service instances in the provided AMF Set.

-	At intra-PLMN mobility, if a target AMF instance needs to be selected, the AMF provides the source AMF Set ID, source AMF Region ID, and the target location information, S-NSSAI(s) of Allowed NSSAI in the request, optionally NRF to use. The SCP will select a target AMF instance belonging to the target AMF set in target AMF Region which can be the mapping of the source AMF set in source AMF region.

-	At inter PLMN mobility, the source AMF selects indicates "roaming" to the SCP. The SCP interacts with the NRF in source PLMN so that the NRF in source PLMN can discover an AMF in the target PLMN via target PLMN NRF.

*** END CHANGES ***