3GPP TSG SA WG2 Meeting #135	S2-1909869

# Split, Croatia, 14-18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1911 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative QoS Profile Handling for V2X Communication Over Uu Reference Point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When using Alternative QoS Profiles, QoS Upgrade is needed as well as downgrade. As Alternative QoS profiles can differ with different parameters (eg PDB and GBR) it is no always easy for the RAN to determine which QoS Profile is ‘better’ from the application’s point of view. Hence we need to say that the Alternatives QoS Profiles have an order. The current text does not indicate what QoS (if any) the RAN offers after sending a Notification to the core network. It is logical however, that the RAN immediately moves to providing the QoS of the indicated Alternative QoS Profile while waiting for the CN to align and the CN to inform the UE via NAS (and N2) signalling. Support of Handover situations is needed for different levels of congestion/capacity in the target RAN node. The UE needs to be informed of GBR being provided by the RAN (the Prioritised Bit Rate parameter in TS 38.331/36.331 does not provide sufficient granularity to do this). Ideally UE impacts should be avoided. Queuing at initial admission needs to be supported to avoid random resource allocation by the RAN when GBR flows that are “not vulnerable to preemption” are commonly used. The initial admission acknowledgement by the RAN must use a single message from the RAN rather than promptly following it with a Notification of QoS downgrade/failure (to avoid, e.g. a car leaving its safe car park space and then being stopped in the middle of a street 2 m later) Support for a mix of upgraded and non-upgraded RAN nodes is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section added to avoid confusion being caused by editing old text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature will be unusable owing to many missing specification aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2a, 5.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * * *

#### 5.7.1.2a	Alternative QoS Profile

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, the Alternative QoS Profile(s) to the NG-RAN.

An Alternative QoS profile represents a combination of QoS parameters and has the same format as the QoS profile for that QoS Flow.

When the NG-RAN sends a notification to the SMF that the QoS profile cannot be guaranteed (or can now be guaranteed again), the NG-RAN shall include a reference to the Alternative QoS profile (as specified in clause 5.7.2.4.2) that the NG-RAN can now guarantee (unless non of them can be guaranteed).

* * * Second change * * * *

#### 5.7.2.4	Notification control

##### 5.7.2.4.1	General

The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR can no longer be guaranteed, NG-RAN shall send a notification towards SMF and may provide values for the GFBR, the PDB and the PER parameters it can guarantee. Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not delivering the requested GFBR for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to guarantee the GFBR again.

NOTE 1:	NG-RAN can decide that the GFBR can no longer be guaranteed based on, e.g. measurements like queuing delay or system load.

.

Upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed, the SMF may forward the notification to the PCF, see TS 23.503 [45]. 5GC may initiate N2 signalling to modify or remove the QoS Flow.

When the NG-RAN determines that the GFBR can be guaranteed again for a QoS Flow (for which a notification that the GFBR can no longer be guaranteed has been sent), the NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed whenever necessary.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the GFBR can be guaranteed again and therefore a frequent signalling of GFBR can be guaranteed again followed by GFBR can no longer be guaranteed is not expected.

NOTE 3:	If the QoS Flow is modified, the NG-RAN restarts the check whether the GFBR can no longer be guaranteed according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification that the GFBR for a QoS Flow can no longer be guaranteed. The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. The accepted QoS Flows are included in the N2 Path Switch Request or N2 Handover Request Acknowledge message from the NG-RAN to the AMF. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that GFBR can be guaranteed again for this QoS Flow. After the handover is successfully completed, the Target NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed for such a QoS Flow whenever necessary. If the SMF has previously notified the PCF that the GFBR can no longer be guaranteed and the SMF does not receive an explicit notification that the GFBR can no longer be guaranteed for that QoS Flow from the Target NG-RAN within a configured time, the SMF shall notify the PCF that the GFBR can be guaranteed again.

##### 5.7.2.4.2	Notification Control with Alternative QoS Profiles

The 5GC may provide the NG-RAN with a list of Alternative QoS Profiles for a GFBR flow with Notification Control. The list is ordered by the 5GC in preference order.

The NG-RAN continuously attempts to deliver the most preferred Alternative QoS Profile.

NOTE 1: 	When Alternative QoS Profiles are in use for a flow, the NG-RAN ignores the legacy QoS Profile for that flow.

Whenever the NG-RAN changes to a better or worse Alternative QoS Profile, the NG-RAN sends a Notification to the Core Network that indicates the new Alternative QoS Profile. Upon sending this Notification, the NG-RAN shall start to fulfill QoS of the indicated Alternative QoS Profile.

If the NG-RAN is unable to even deliver the least preferred Alternative QoS Profile, then the NG-RAN sends a Notification to the 5GC indicating this situation.

NOTE 2:	The NG-RAN is expected to apply some hysteresis to changes of Alternative QoS Profiles.

Unless the SMF is certain that the UE does not need to know about changes in the GBR that the NG-RAN is providing, upon receipt of a Notification indicating a change of Alternative QoS Profile, the SMF shall inform the UE of the new GBR [using the Rel 15 N1 interface Session Modification that is embedded in N2 PDU Session Request signalling]

At Xn/N2 handover, the target RAN node (shall be informed of the ordered list of Alternative QoS Profiles and) may need to change to a worse Alternative QoS Profile or be able to change to a better Alternative QoS Profile. If the Alternative QoS Profile is changed, then the NG-RAN informs the 5GC of the new Alternative QoS Profile along with (for Xn handover) the Path Switch Request or (for N2 handover), the Handover Notify.

At X2/N2 handover, if the Target NG-RAN node is unable to even deliver the least preferred Alternative QoS Profile, then the NG-RAN releases that flow (which causes the Traffic Flow Template in the UE to be removed) and informs the 5GC of the release in the (for Xn handover) Path Switch Request or (for N2 handover), the Handover Notify.

At handover into a congested cell, removal of the TFT in the UE can be avoided if the GFBR is set to 0 bit/s in the least preferred Alternative QoS Profile.

At initial establishment of the GFBR flow, the 5GC can request that the establishment is queued in a congested NG-RAN node by setting the GFBR of the least preferred Alternative QoS Profile to 0kbit/s.

At initial establishment of the GFBR, the NG-RAN shall use a single message to acknowledge the establishment of the flow and the Alternative QoS Profile that is now in use.

NOTE 3: 	The use of an acknowledgement promptly followed by a notification of a change of Alternative QoS Profile is undesirable as the initial acknowledgement can trigger actions that assume the most preferred QoS profile is being fulfilled.

To handle a mixture of NG-RAN nodes that support and do not non-support Alternative QoS Profile handling, a supporting target RAN node shall send an indication of its support to each of the UE’s SMFs in the Path Switch Request and Handover Notify messages. Following mobility through a non-supporting part of the RAN to a supporting part of the NG-RAN, an SMF wishing to use Alternative QoS Profiles can then resend this information to the NG-RAN.

NOTE 4:	Homogeneous RAN support is desirable.

* * * End of changes * * * *