SA WG2 Meeting #S2-135	S2-1909897

14 - 18 October, 2019, Split Croatia

Source:	Apple

Title:	Solution for paging collision avoidance

Document for:	Discussion / Approval

Agenda Item:	8.4

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes a new solution for paging collision avoidance.

# 1	Background

Proposes new solution for paging collision avoidance for Dual-USIM device.

* * * * Start of Change * * * *

## 6.X	Solution #X: Paging collision avoidance

### 6.X.1	Introduction

Editor's note:	This clause lists the key issue(s) addressed by this solution.

The solution applies to Key Issue #X "Mechanism for paging collision avoidance".

The solution applies to both 5GS and EPS.

### 6.X.2	Functional Description

Editor's note:	This clause outlines solution principles and documents any assumptions made.

The solution is based on including a capability indication for the MUSIM feature in the registration procedure.

The UE may provide an option to the user to dynamically enable or disable the secondary USIM. In such a scenario, when the UE changes from a Single USIM to MUSIM mode, or vice versa, the UE updates its capability for the MUSIM feature in a re-registration procedure.

The network can use the MUSIM capability information to increase the timer used for retrying the paging procedure or extend the number of retry attempts.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Existing registration procedure applies as specified in TS 23.502 clause 4.2.2.2.

### 6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

For 5GS:

UE and AMF need to be able to handle MUSIM capability indication in registration procedure.

For EPS:

UE and MME need to be able to handle MUSIM capability indication in attach procedure.

### 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Change * * * *