3GPP TSG SA WG2 Meeting #135	S2-1910008

# Split, Croatia, October 14 - 18, 2019	(Revision of S2-1909268)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1786 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Alternative QoS Profiles at handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Vodafone? Fraunhofer HHI? Volkswagen AG? Ericsson? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to RAN3 response R3-194795: "1) QoS Upgrading amongst the set of Alternative QoS Profiles is feasible from a RAN3 perspective. 2) handover aspects may need further consideration. The current concept of the CN being in control of all QoS changes may not currently work well for handover as it is likely to lead to the GBR flow being released by a congested cell. The flow’s RAN-priority would then be dropped from “flow maintenance” to “new flow” (which is likely to lead to a long interruption time in the congested cell)." |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added multi-QoS treatment support in the handover case, which includes: - - provision of the prioritized list of Alterntive QoS Profiles to the target NG-RAN; - - admission control using Alterntaive QoS Profile(s) at the target NG-RAN; - notification of selected Alternative QoS Profile to the SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Alternative QoS Profiles are not supported at handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * * *

#### 5.7.1.2a	Alternative QoS Profile

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, the Alternative QoS Profile(s) to the NG-RAN.

An Alternative QoS profile represents a combination of QoS parameters and has the same format as the QoS profile for that QoS Flow.

When the NG-RAN sends a notification to the SMF that the QoS profile cannot be guaranteed, the NG-RAN may include the reference to the Alternative QoS profile(s) (as specified in clause 5.7.2.4) to indicate the QoS that the NG-RAN can guarantee.

During handover, the prioritized list of Alternative QoS Profile(s) is provided to the target NG-RAN. Before rejecting a QoS Flow during admission control, the target NG-RAN shall check whether the values of the GFBR, the PDB and the PER parameters that the target NG-RAN can fulfil match any of the Alternative QoS Profile(s) in the indicated priority order. If there is a match, the target NG-RAN shall accept the QoS Flow and indicate the reference to the matching Alternative QoS Profile with the highest priority to the source NG-RAN.

NOTE x:	The handover can continue with a target NG-RAN that does not accept the values in the QoS profile but only the values in one of the Alternative QoS Profiles.

If the handover continues, the target NG-RAN shall indicate to the SMF the reference to the Alternative QoS Profile it has accepted and which the NG-RAN currently fulfils. In addition, the target NG-RAN should try to fulfil the GFBR, the PDB and the PER of the QoS profile and when the target NG-RAN determines that this is possible (e.g. other GBR QoS Flows have been released or handed over), the target NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again (see clause 5.7.2.4). If the target NG-RAN determines that the GFBR, the PDB or the PER of the indicated Alternative QoS Profile are no longer fulfilled, NG-RAN shall send a notification towards SMF according to the principles described in clause 5.7.2.4.

* * * End of changes * * * *