3GPP TSG-SA2 Meeting #135	S2-1910215

# Split, Croatia, 14th October 2019 - 18th October 2019				   (revision of S2-1909764)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1849 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SMF identifier in HR roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) In TS 23.502 Rel-15, in the HR PDU Session establishment procedure, after the AMF has discovered the H-SMF and V-SMF, the AMF provides the “identity of the H-SMF” to the V-SMF in the Nsmf_PDUSession_CreateSMContext Request service operation. See step 3a in 4.3.2.2.2 in TS 23.502: …the AMF also provides the identity of the H-SMF it has selected in step 2… Stage 2 ”the identity of the H-SMF” is implemented as “the URI of the Nsmf_PDUSession service of the H-SMF” (hSmfUri) in Rel-15 Stage 3 (TS 29.502). In TS 23.502 Rel-16, for establishment of HR PDU sessions or PDU sessions with an I-SMF with delegated discovery, Annex E requires the AMF to provide the selected “SMF ID” in the Create SM Context Request, to be forwarded by the V-SMF or I-SMF towards the downstream SCP as a discovery and selection parameter. The term “SMF ID” has been misleading for stage 3. In C4-193735 (CR 29.502 #0169) the term “SMF ID” has been interpreted as a need of including a new smfId attribute in the Create SM Context Request to convey the SMF Instance ID for HR PDU sessions or PDU sessions with an I-SMF, which would create backward compatibility issues with Rel-15. 2) In the procedure described in Annex E.2, steps 2-3 are optional but the following cases A or B assume that a NRF discovery response was received in step 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) In the context of HR PDU sessions or PDU sessions with an I-SMF with delegated discovery, replace the term “SMF ID” with “the endpoint (e.g. URI) of the selected H-SMF”, to clarify that it is the same parameter as used in Rel-15, i.e. the URI of the H-SMF like in the case without delegated discovery. Similar change is applied to PDU session establishment with I-SMF. 2) Clarify that the NRF discovery response is optional and the discovery may be performed also by the SCP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) The use of the term “SMF ID” has been interpreted by stage 3 as a new parameter in comparison to Rel-15, therefore creating a backward compatibility issue. 2) Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.1, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

# E.1	Delegated SMF discovery in the Home Routed scenario

Figure E.1: Delegated Discovery of SMF in the Home Routed Scenario

1.	The AMF sends Nnrf_NFDiscovery Request to the V-NRF. The AMF may indicate the maximum number of H-SMF instances to be returned by the NRF.

2.	The NRF in VPLMN and NRF in HPLMN interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

3.	The AMF gets Nnrf_NFDiscovery service response with one or more profile(s) of SMF(s) in HPLMN.

4.	The AMF selects an SMF instance in HPLMN (H-SMFendpoint).

5.	The AMF builds a Nsmf_PDUSession_CreateSMContext Request that includes the endpoint (e.g. URI) of the selected H-SMF in the body of the request. If the AMF supports delegated SMF discovery and is configured to apply it, the AMF sends the Nsmf_PDUSession_CreateSMContext Request with Discovery & Selection parameters to the selected SCP in the VPLMN. Discovery & Selection parameters include S-NSSAI, UE location (TAI), i.e. parameter for V-SMF selection.

6.	[Optional] The SCP in VPLMN sends Nnrf_NFDiscovery Request to the V-NRF using Discovery & Selection parameters received from AMF.

7.	[Optional] The SCP in VPLMN gets Nnrf_NFDiscovery service response with profile(s) of SMF(s) in VPLMN.

8.	The SCP in VPLMN selects an SMF instance in VPLMN (V-SMF), which supports the Discovery & Selection parameters received earlier from the AMF.

9.	The SCP in VPLMN forwards the Nsmf_PDUSession_CreateSMContext Request received from the AMF to the selected SMF instance in VPLMN.

10.	If the V-SMF does not support delegated SMF discovery or is not configured to apply it (Case A), the V-SMF sends Nsmf_PDUSession_Create Request directly to the H-SMF. Otherwise (Case B), the V-SMF sends the Nsmf_PDUSession_Create Request to the SCP in VPLMN but adds Discovery & Selection parameter set to H-SMF endpoint received from the AMF. In both cases, the V-SMF uses the received endpoint (e.g. URI) of the selected H-SMF to construct the target destination to be addressed.

NOTE:	The Nsmf_PDUSession_Create Request sent by the V-SMF in Case A and in Case B is the same apart from the Discovery & Selection parameter. The Nsmf_PDUSession_Create Request received by the H-SMF in Case A and in Case B is the same.

11.	The SCP in VPLMN sends a Nsmf_PDUSession_Create Request to the selected SMF instance in HPLMN indicated in step 10.

When the V-SMF responds to AMF with Nsmf_PDUSession_CreateSMContext Response as in clause 4.3.2.2.2 in step 3b, if the AMF has not stored the SMF Service Area for the V-SMF, the AMF shall obtain the SMF Service Area for the concerned V-SMF from the NRF using the Nnrf_NFManagement_NFStatusSubscribe service operation.

* * * * Start of 2nd Change * * * *

# E.2	Delegated I-SMF discovery

Figure E.2: Delegated Discovery of I-SMF

The following impacts are applicable to clause 4.23.5 (PDU Session Establishment procedure) to support delegated SMF discovery:

1.	If the AMF supports delegated SMF discovery and is configured to apply it, the AMF sends an Nsmf_PDUSession_CreateSMContext Request together with discovery and selection parameters to a SCP. The discovery and selection parameters include S-NSSAI, DNN, TAI that corresponds to the UE locationrequired SMF capability (e.g. support of ATSSS).

2.	[Optional] The SCP sends an Nnrf_NFDiscovery Request to the NRF. The request includes discovery and selection parameters received from AMF in step 1.

3.	[Optional] The SCP gets Nnrf_NFDiscovery service response. The response may include one or more profile(s) of SMF(s).

Depending on the available information , the SCP may either execute steps in Case A or in Case B.

Case A There are, either in the NRF response or discovered by the SCP, one or more SMF instances that support Discovery and selection crireria set by the AMF.

4.	The SCP selects an SMF instance.

5.	The SCP forwards the Nsmf_PDUSession_CreateSMContext Request to the selected SMF instance.

Case B There is, either in the NRF response or discovered by the SCP, no SMF instance that supports Discovery and selection crireria set by the AMF.

6.	The SCP returns an Nsmf_PDUSession_CreateSMContext Response to the AMF with an error 'NF not found'

7.	The AMF sends Nnrf_NFDiscovery Request to the NRF. The AMF may indicate the maximum number of SMF instances to be returned by the NRF.

8.	The AMF gets Nnrf_NFDiscovery service response with one or more profile(s) of SMF(s).

9.	The AMF selects an SMF instance endpoint.

10.	The AMF builds a Nsmf_PDUSession_CreateSMContext Request that contains the endpoint (e.g. URI) of the selected SMF in the body of the request. If the AMF supports delegated SMF discovery and is configured to apply it, the AMF sends the Nsmf_PDUSession_CreateSMContext Request to a SCP together with Discovery and selection parameters that include S-NSSAI, TAI that corresponds to the UE location, i.e. parameter for I-SMF selection.

11.	[Optional] The SCP sends an Nnrf_NFDiscovery Request to the NRF. The request includes Discovery and selection parameters received from AMF (including the TAI that corresponds to the UE location).

12.	[Optional] The SCP gets Nnrf_NFDiscovery service response. The response may include one or more profile(s) of I-SMF(s).

13.	The SCP selects an I-SMF instance that supports the TAI.

14.	The SCP forwards the Nsmf_PDUSession_CreateSMContext Request received from the AMF to the selected I-SMF instance.

15.	If the I-SMF does not support delegated SMF discovery or is not configured to apply it (Case A), the I-SMF sends Nsmf_PDUSession_Create Request directly to the SMF. Otherwise (Case B), the I-SMF sends the Nsmf_PDUSession_Create Request to the SCP but adds Discovery & Selection parameter set to the SMF endpoint received from AMF. In both cases the I-SMF uses the received endpoint (e.g. URI) of the selected SMF to construct the target destination to be addressed.

NOTE:	The Nsmf_PDUSession_Create Request sent by the I-SMF in Case A and in Case B is the same apart from the Discovery & Selection parameter. The Nsmf_PDUSession_Create Request received by the SMF in Case A and in Case B is the same.

16.	The SCP forwards the Nsmf_PDUSession_Create Request to the selected SMF instance indicated in step 15.

The procedure continues as described in clause 4.3.2.2.2 from Step 7.

Difference comparing to procedure defined in clause 4.3.2.2.2, the V-SMF and V-UPF are replaced by I-SMF and I-UPF, and H-SMF and H-UPF are replaced by SMF and UPF(PSA) respectively. Also only the S-NSSAI with the value defined by the serving PLMN is sent to the SMF.

When the I-SMF responds to AMF with Nsmf_PDUSession_CreateSMContext Response as in clause 4.3.2.2.2 in Step 3b, if the AMF has not stored the SMF Service Area for the I-SMF, the AMF shall obtain the SMF Service Area for the concerned I-SMF from the NRF using the Nnrf_NFManagement_NFStatusSubscribe service operation.

* * * * End of Changes * * * *