SA WG2 Meeting #135 	S2-1910274

# October 14-18th, 2019 ; Split, HR		                               			  (revision of S2-1909019)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1714 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (I)SMF notifications: which SMF events need to be supported by ISMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-09-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF events are defined as The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1): - UE IP address / Prefix change: - PDU Session Release. - UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed. - Change of Access Type; The event notification contains the new Access Type for the PDU Session. - PLMN change; The event notification contains the new PLMN Identifier for the PDU Session. - Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including: - First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time. - First downlink packet per source of the downlink IP traffic discarded. - First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s). SMF is aware of following events - UE IP address / Prefix change - PDU Session Release. - UP path change: This corresponds to N16a notifications described in step 4 of Figure 4.23.9.1 - PLMN change: A PLMN change should correspond to a VSMF change, insertion or release and the VSMF provides its PLMN Id in PduSessionCreateData (29.502) sent over N16 So the underlined events remain. Event consumers should not have to know about ISMF; they subscribe onto SMF Lets’ consider first “First downlink packet per source of the downlink IP traffic” (buffered : discarded : transmitted) This belongs to VSMF/ISMF responsibility as VSMF/ISMF is responsible of the transitions between UP ACTIVE and UP INACTIVE (IDLE-CONNECTED) Lets’ consider now the “Change of Access Type”; This is more tricky: - The change of Access type may be within a I-SMF or may correspond to the insertion / change / removal of an I-SMF - ATSSS adds a specific case where the Access type may be 3GPP, N3GPP or both (ATSSS) - In HR scenarios (with different PLMN(s) per access) each I-SMF/V-SMF may see only one access type while the PDU Session is actually in ATSSS mode - So it looks better to keep the SMF responsible of related notifications as the VSMF / ISMF may only have partial view |  |  |  |  |  |  |  |  |
| Summary of change: |  | Consumers of SMF events do not need to be aware of the insertion / removal / change of an I-SMF as they subscribe to the SMF of the PDU Session. Except for the events documented in the new clause 4.23.X, the I-SMF does not need to support the events defined in 23.502 [2] clause 5.2.8.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for consumers to subscribe to SMF events involving I-SMF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all text is new)

### 4.23.X          SMF event  exposure service

Consumers of SMF events do not need to be aware of the insertion / removal / change of an I-SMF as they always subscribe to the SMF of the PDU Session.

Except for the events documented in the present clause, the I-SMF does not need to support the events defined in clause 5.2.8.3.1.

For Events “First downlink packet per source of the downlink IP traffic (buffered / discarded / transmitted)”,  when an I-SMF is involved in the PDU Session, the SMF subscribes / unsubscribes onto I-SMF on behalf of the event consumer (e.g. at I-SMF insertion or when a consumer subscribes or un subscribes while an I-SMF serves the PDU Session) and the I-SMF directly notifies the event consumer. At I-SMF change, the source I-SMF provides the related event subscriptions to the target I-SMF.

For the “Change of Access Type” event,

- the SMF is responsible of notifying the event consumers,

NOTE:	in case of a MA PDU in some deployment cases, an I-SMF or a V-SMF cannot provide an accurate information as it can only be aware of one access of the MA PDU Session

- In cases where the SMF cannot otherwise be aware of the access type of the PDU Session, the SMF may subscribe as an event consumer into the I-SMF about the access type of the PDU Session.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES