SA WG2 Meeting #135 	S2-1910290

# October 14-18th, 2019 ; Split, HR		                               			  (revision of S2-1909886)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1865 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Relation between mobility events and control local offload in ETSUN cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-09-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ISMF addition / removal / change (as AMF change) are done as part of mobility procedures. In R15 and in R16 for non-ETSUN cases addition / removal / change of ULCL and PSAs are done as part of procedures (defined in § 4.3.5) that are separate from the mobility procedures. It is not obvious that for ETSUN we should deviate from the principles used so far Even, considering current clause and Figure 4.23.9a-1 there are questions: - Step 1 reads “The AMF decides to change the I-SMF e.g. due to UE mobility” without specifying which mobility event - Thus the relation between clause and Figure 4.23.9a-1 and the different mobility procedures is not clearly defined - Solving this issue for clause 4.23.9a would imply a high combination of cases to document and thus a high number of procedures to add: addition of I-SMF and local traffic offload for - X2 HO - N2 HO - Service request - Mobility registration - Mobility between 3GPP and Non 3GPP access - Mobility from EPS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.23.9a is voided |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 4.23.9a contradicts the rest of the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.23.9a	void

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES