SA WG2 Meeting #135	S2-1910331

# 14 - 18 Oct 2019, Split, Croatia	 										(revision of S2-1909449)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1819 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network selection correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Specification requirements should not be duplicated, especially in TSs that are maintained by different groups. In the interaction of network sharing and PLMN selection, the situation is actually worse, as the requirement in TS 23.501 is in conflict with the PLMN selection requirements already specified in TS 22.011 and TS 23.122. TS 23.122 specifies that re-selection to shared network cell does not trigger PLMN selection, so the UE must remain in the RPLMN during cell re-selection. However, other reasons for PLMN selection might still apply after the cell re-selection, thus the CT1 requirement leaves the potential PLMN change after the re-selection open if e.g. background scanning finds a higher priority PLMN. TS 23.122 clause 4.4.3 specifies the following: “When the MS reselects to a cell in a shared network…The MS shall choose one of these PLMNs. If the registered PLMN is available among these PLMNs, the MS shall not choose a different PLMN.” The above requirement quite correctly directs the UE to stay in the RPLMN at cell re-selection, but it also leaves PLMN change possible afterwards, if it is necessary due to other existing UE requirements. The statement “UE should not change to another PLMN as long as the selected PLMN is available” in TS 23.501 clause 5.18.3 is not correct, as it would prevent PLMN change for any reason. This violates background scanning and manual PLMN selection requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Superfluous text is removed and it is left for TS 23.122 to specify the PLMN selection requirements for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background scanning and manual PLMN selection are not supported in shared network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### ***** First change *****

### 5.18.3	Network selection by the UE

NOTE:	This clause applies to UEs not operating in SNPN access mode. Network selection for UEs set to operate in SNPN access mode is described in clause 5.30.2.4.

A UE that has a subscription to one of the sharing core network operators shall be able to select this core network operator while within the coverage area of the shared network and to receive subscribed services from that core network operator.

Each cell in shared NG-RAN shall in the broadcast system information include the PLMN-IDs concerning available core network operators in the shared network.

When a UE performs an Initial Registration to a network, one of available PLMNs shall be selected to serve the UE. UE uses all the received broadcast PLMN-IDs in its PLMN (re)selection processes which is specified in TS 23.122 [17]. UE shall inform the NG-RAN of the selected PLMN so that the NG-RAN can route correctly. The NG-RAN shall inform the core network of the selected PLMN.

As per any network, after Initial Registration to the shared network and while remaining served by the shared network, the network selection procedures specified in TS 23.122 [17] may cause the UE to perform a reselection of another available PLMN.

UE uses all of the received broadcast PLMN-IDs in its cell and PLMN (re)selection processes.