SA WG2 Meeting #135	S2-1910467

Oct 14 - 18, 2019, Split, Croatia	(was S2- S2-1908793)

Source:	vivo, Huawei, OPPO, ZTE, China Mobile, China Unicom

Title:	Key issue# X: Service collision and service interruption

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This paper proposes the Key issue# X: Service collision and service interruption among the services associated with different USIMs n for TR 23.761.

- Introduction

For a MUSIM (e.g. USIM_A and USIM_B) UE with single Tx, cannot support services transmission over different USIM simultaneously, may suffer from the following issues:

| No. | Connection associated with USIM A | Connection associated with USIM B | Issues |
| --- | --- | --- | --- |
| 1 | RRC connection setup/resume requirement due to buffered UL data or paging response requirement | RRC connection setup/resume requirement due to buffered UL data or paging response requirement | Services collision, which RRC connection setup/resume firstly? |
| 2 | Connected mode + Ongoing services, e.g. gaming, voice, emergency call | Idle mode + DL paging reception, paging response, data transmission | Services interruption or service delay increases too long |

Case1: UE’s connections associated with each USIM are in idle mode. Because of either buffered UL data or paging response requirement, RRC connection setup/resume are required for both USIM, the service collision happens.

Case2: When the connection associated with USIM_A is in connected mode and has ongoing service, UE may receive paging message associated with USIM_B. Responding to the page associated with USIM_B will make the connection associated with USIM_B active but make the connection associated with USIM_A interrupted. That will result in service interruption of USIM_A or delay greatly the ongoing services of USIM_A.

- If the ongoing service is voice (e.g. the UE is 1Tx/2Rx), the voice is interrupted

- If the connection of USIM A is being used to transmit data for a gaming service, then the gaming service experience becomes unsmooth or interrupted, depending on how long the connection for USIM_A is being interrupted. So far, unsmooth gaming experience and gaming interruption have become a critical compliant from users.

- Furthermore, in TS 22.834:

  - [CPR.8.3-2]: The MUSIM UE shall avoid interruptions of emergency call procedures of one USIM by other services of other USIM.

  - [CPR.8.2-6]: The 3GPP system shall be able to minimize the impacts of services associated to one USIM for services associated to another USIM for a MUSIM UE.

  - [CPR.8.2-8]: The 3GPP system shall be able to allow a MUSIM UE to provide voice from one USIM and data services from another USIM simultaneously.

Figure-1: Example for case2

2	Proposal

It is proposed to including following key issue into the TR 23.761.

*** Start of changes ***

5	Key Issues

5.X	Key Issue X: < Service collision and service interruption with Multi-USIM device >

5.X.1	Description

When a Multi-USIM device is in an active communication associated with USIM A, it will suffer from collision or interruption to the ongoing services associated with USIM A when the Dual-USIM device is required to activate the communication associated with USIM B.

This key issue aims at studying to solve service collision and service interruption among the services associated with different USIMs. Solutions to this key issue are expected to address the following aspects:

-	 Investigate that when a Multi-USIM device is in an active communication associated with USIM A, how to avoid unnecessary interruptions to USIM A when there is a MT service request for USIM B.

*** End of changes ***