SA WG2 Meeting #S2-135	S2- 1910624

14 - 18 October, 2019, Split Croatia                                            (revision of S2-1909395)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1802 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSN 5QI calrfication and static TSC QoS Flow establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 5.27.3, Time Sensitive Communication is calrified to use QoS Flows with Delay Critical GBR resource type, standardized 5QIs, but these are not enough to serving TSN traffic classes. Till now TS 23.501 has specified 4 standardized 5QIs for Delay Critical GBR resource type, while upto 8 traffic classes with different priority and bridge delay may be applied for TSN traffic (each traffic class may be mapped into a QoS Flow in the PDU session). TSN streams have varity of traffic characteristics (e.g. frame size, packet delay, priority etc.), the standardized 5QIs may not fit all of them. For the above two reasons, pre-configured 5QIs and dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) may required to adapt to different TSN traffic characteristics. In a wireline based TSN system, the connectivity between TSN nodes (Bridges/End-stations) are normally static. A TSN-5G bridge should be able to emulate the behaviour of a normal TSN bridge. Inside the TSN-5G bridge, static connections between a DS-TT and a NW-TT might be required, including the PDU sessions, QoS flows, and their mapping with TSN traffic classes. The pre-configure TSN QoS Flows which are established at the beginning of PDU Session Establishement can simplify the QoS negotiation and mapping process with TSN-AF and CNC, reduce the signaling exchange for session management and TSN QoS configuration. The introduction of static configuration of QoS flow can avoid the complexity of dynamic TSN-QoS control and minimize the impact to CNC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper clarifies to extend the 5QI of TSC QoS Flows from standardized 5QIs to pre-configured 5QIs and dynamically assigned 5QI values as well. This paper clarifies the usage of always-on PDU Session for TSN, the establishment of pre-configured TSN QoS Flows and the mapping with traffic classes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the TSN-5GS QoS mapping. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.3, 5.28.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

*** BEGIN CHANGES ***

### 5.27.3	Support for TSC QoS Flows

TSC QoS Flows use a Delay Critical GBR resource type and TSC Assistance Information. TSC QoS Flows may use standardized 5QIs, pre-configured 5QIs or dynamically assigned 5QI values (which requires signalling of QoS characteristics as part of the QoS profile) as specified in clause 5.7.2. Within each Period (defined by periodicity), TSC QoS Flows are required to transmit only one burst of maximum size MDBV within the AN-PDB. Known QoS Flow traffic characteristics provided in the TSCAI may be used to optimize scheduling in the 5GS.

The following is applicable for the 5QI defined for TSC QoS Flows:

1.	The TSCAI Burst Size may be used to set the MDBV.

2.	The PDB is explicitly divided into 5G-AN PDB and CN PDB. The 5G-AN PDB is the packet delay budget applicable to the radio interface, including AN processing. The CN PDB is the delay between a UPF terminating N6 and a 5G-AN. Separate delay budgets are necessary for calculation of expected packet transmit times on 5G System interfaces.

3.	The TSCAI Burst Arrival Time calculation uses UE-DS-TT residence time and CN PDB as per clause 5.27.5. For DL TSCAI Burst Arrival Time determination, CN PDB is needed. For UL TSCAI Burst Arrival Time determination, UE-DS-TT residence time are needed.

*** NEXT CHANGE ***

*** NEXT CHANGE ***

### 5.28.4	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.28.1, and (2) QoS configuration phase as in clause 5.28.2

(1)	The TSN Translator AF shall be pre-configured via OAM the mapping table and represents what is being reported or exposed to the TSN system (i.e. the CNC) through the relevant TSN information objects, such as the Traffic Class Table for every port, see the IEEE 802.1Q [X] and IEEE 802.1Qcc [95].

(2)	CNC distributes the TSN QoS requirements and TSN scheduling parameters (specific for current node) to 5G virtual bridge via TSN AF. Alternatively, the 5GS virtual bridge may pre-request or query CNC for the TSN QoS and traffic information.

The PCF mapping table provides a mapping from TSN QoS information to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS flow or use the pre-configured 5QI for 5G QoS Flow for the requested traffic class according to the selected QoS policies from the TSN AF traffic requirements.

Figure 5.28.4-1 illustrates the functional distribution of the mapping tables.

Figure 5.28.4-1: QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements in the 5GS are: traffic classes and their priorities per port, bridge delays per port pair and traffic class (independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), and propagation delay per port (txPropagationDelay).

QoS mapping table between port traffic classes and 5QI should be matching the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles.

Once the 5QIs to be used for TSN streams are identified, then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge per port and per traffic class. The most relevant information received is the scheduling for every traffic class and port of the bridge. At this point, it is possible to retrieve the real QoS requirements by identifying the traffic class of the port. Then the traffic class to 5QI mapping can be performed using the QoS mapping table in the TSN AF. Subsequently, the TSC QoS flow can be configured using the 5QI retrieved from the QoS mapping table. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS. The scheduling configuration information per traffic class is mapped to trigger creation/modification of a QoS flow in 5GS.

*** END CHANGES ***