SA WG2 Meeting #S2-135	S2-1910817

14 - 18 October, 2019, Split Croatia	(e-mail revision 2 of S2-1910690)

Source:	Intel, China Telecom, Motorola Mobility, Lenovo, Apple, NEC, Samsung, NEC

Title:	Key issue on mechanism for suspension / resumption

Document for:	Discussion / Approval

Agenda Item:	8.4

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes a new Key Issue on mechanism for suspension / resumption.

# 1	Background

Proposes new Key Issue on mechanism for suspension / resumption.

* * * * Start of Change * * * *

## 5.X	Key Issue X: Coordinated leaving for Multi-USIM device

### 5.X.1	Description

Consider a Multi-USIM device that has concurrent registrations associated with several USIMs. While actively communicating with the system associated with one USIM (the “current system”), the Multi-USIM device determines that it needs to perform some activity in the system associated with another USIM (e.g. respond to a page, or perform mobility update).

Today, in the absence of any procedure for notifying the network the Multi-USIM device may autonomously leave or release the RRC connection with the current system. This is likely to be interpreted as an error case by the current system and has the potential to distort the statistics in the current system, and misguide the algorithms that rely on them. Moreover, during the Multi-USIM device’s absence from the current system, if the UE cannot receive downlink data or process the paging from the current system, it may result in waste of resources.

The present key issue shall study:

-	How to enable a Multi-USIM device to leave the current 3GPP system in coordination with the network while avoiding wasting the network resource during the leave.

-	How the network handles MT data or MT control-plane activity occurring when Multi-USIM device has left?

NOTE 1:	Any privacy implications of implicitely indicating to the MNO owning one USIM that the UE is also using another USIM (potentially owned by another MNO) will be studied by SA WG3.

NOTE 2:	During the study of this key issue, coordination with RAN WGs will be handled, if needed.

* * * * End of Change * * * *