3GPP TSG-SA2 Meeting #136AH	S2-2000271

# Incheon,South Korea, 13th – 17th January 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1975 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Enhanced Coverage Restriction Control via NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Convida Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the LS received from CT4 (S2-2000009/ C4-194530), it is required to separate the services for UE subscription data retrieval and UE subscription data update. Retrieval UE subscription data and provision UE subscription data have different view. From the subscription data retrieval view (see 23.502 clause 4.27.1 step 3 and 7), subset of UE’s Enhanced coverage restriction data(per user per PLMN) is enough on demand, but from the parameters provision view, the overall picture of UE’s Enhanced coverage restriction data can be retrieved and configured, hence different resources are suggested to use for higher efficiency. And next, AMF and AF belong to two different network domain, separate services are suggested to use for data isolation, and UDM have already defined separated services for them, SDM service is provided to retrieve UE subscription data, PP service is provided for provision of UE subscription data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification that separate services are used to retrieve UE subscritpion data i.e., SDM service and to provision UE subscription data i.e., PP service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of service operations for Enhanced Coverage Restiction Control is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.3.7, 5.2.3.6.2, 5.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of changes ***

##### 5.2.3.3.7	Nudm_SDM_Update service operation

Service operation name: Nudm_SDM_Update

Description: The consumer requests to update the subscription data.

Inputs (required): GPSI, AF Identifier, Transaction Reference ID(s).

Inputs (optional):None.

Outputs (required): Transaction Reference ID(s), Operation execution result indication

Outputs (optional): Transaction specific parameters, if available.

# *** Next change ***

##### 5.2.3.6.2	Nudm_ParameterProvision_Update service operation

Service operation name: Nudm_ParameterProvision_Update.

Description: The consumer updates the UE related information (e.g., Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, Enhanced Coverage Restriction Information) or 5G VN group related information (5G VN group data, 5G VN membership management).

Inputs (required): AF ID, Transaction Reference ID(s).

Inputs (optional): GPSI, External Group ID, at least one of the Expected UE Behaviour parameters or at least one of the Network Configuration parameters or 5G VN group related information, Validity Time or Location Privacy Indication parameters.

Outputs (required): Transaction Reference ID(s), Operation execution result indication.

Outputs (optional): Transaction specific parameters, if available.

# *** Next change ***

## 4.27	Procedures for Enhanced Coverage Restriction Control via NEF

### 4.27.1	General

The support for Enhanced Coverage Restriction Control via NEF enables AF to query status of Enhanced Coverage Restriction or enable/disable Enhanced Coverage Restriction per individual UEs. Figure 4.27.1-1 shows the procedure for Enhanced Coverage Restriction Control via NEF.

Figure 4.27.1-1: Enhanced Coverage Restriction Control via NEF

1.	The AF may enable or disable Enhanced Coverage Restriction or query the status of Enhanced Coverage Restriction by sending the Nnef_ECRestriction_Update Request or Nnef_ECRestriction_Get Request respectively. Both the service operations require GPSI and AF Identifier as required input.

2.	Based on operator policies, if the AF is not authorized to perform the request (e.g. if the SLA does not allow for it) or if the AF has exceeded its quota or rate of submitting Enhanced Coverage Requests, the NEF performs Step 8 and provides a cause value appropriately indicating the failure result.

3.	The NEF sends the Nudm_ParameterProvision_Update Request to update the subscription data for Enhanced Coverage Restriction. The NEF sends the Nudm_SDM_Get Request service operation to query the status of Enhanced Coverage Restriction.

4.	The UDM checks the GPSI and examines whether any included parameters are in the range acceptable by the operator, whether Enhanced Coverage Restriction is supported by the serving NF (i.e., AMF in this case). If this check fails, the UDM provides a cause value indicating the reason for failure condition to the NEF in step 7

In the case of Nudm_ParameterProvision_Update Request, the UDM sets the Enhanced Coverage Restriction information to the appropriate value and procedure continues to step 5.

In the case of Nudm_SDM_Get Request, the UDM may retrieve the status of Enhanced Coverage Restriction information from UDR using Nudr_DM_Query Request and skip steps 5 and 6.

5.	The UDM sends Nudm_SDM_Notification and provide AMF with updates Enhanced Coverage Restriction information.

6.	The AMF updates the Enhanced Coverage Restriction information stored in the UE context. The AMF will transfer Enhanced Coverage Restriction information stored as part of its UE context during AMF change.

NOTE:	UE is informed of the updated Enhanced Coverage Restriction information at the next Registration procedure or based on the local policy the network can de-register the UE indicating re-registration is required.

7.	The UDM sends the Nudm_ParameterProvision_Update Response or Nudm_SDM_Get Response to the NEF.

8.	The NEF sends the Nnef_ECRestriction_Update Response or Nnef_ECRestriction_Get Response to the AF.

# *** End of changes ***