3GPP TSG-WG SA2 Meeting #136AH 	S2-2000410

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0073 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR PC5 user plane protocol stack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the data arrives the V2X layer has to determine the PFI based on the application input, which has been described in clause 5.4.1.1.3： - it maps the V2X data packets to PC5 QoS flows - select PFI for all the packets and send it to AS layer However, in the current design of user plane protocol stack, the V2x layer is not reflected in the user plane, as shown in the Figure 6.1.1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figure 6.1.1-1 is updated to cleary point out that the IP and Non-IP layers correspond to the V2X layer. A new sentence has been introduced to describe the handling of IP/Non-IP packets before they are passed to the AS layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user plane protocol is not reflecting the design of user plane handling of V2X pacekts. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.1.1	User plane for NR PC5 reference point supporting V2X services

Legend:

-	PC5-U: The SDAP/PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [11].

-	For PDCP SDU type "Non-IP", a "Non-IP Type" header included in the SDU by upper layer to indicate the type of non-IP messages carried will be specified in stage 3 specification.

Figure 6.1.1-1: User Plane for PC5 interface

IP and Non-IP PDCP SDU types are supported for the V2X communication over PC5.

For IP PDCP SDU type, only IPv6 is supported. The IP address allocation and configuration are as defined in clause 5.6.1.1.

The Non-IP PDCP SDU contains a Non-IP Type header, which indicates the V2X message family used by the application layer, e.g. IEEE 1609 family's WSMP [18], ISO defined FNTP [19].

NOTE:	The Non-IP Type header and allowed values will be defined in stage 3 specification.

The IP/Non-IP packet is handled by the V2X layer before transmitting it to the AS layer, e.g., selecting the corresponding source and destination Layer-2 ID and marking the correct PFI.

# * * * * End of changes * * * *