SA WG2 Meeting #136AH	S2-2001208

Incheon, Korean, January 12th, 2020	revision of S2-2000792

Source:	China Mobile, Huawei, AsiaInfo, NEC

Title:	key issue: NWDAF-assisted charging information generation

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA / Rel-17

Abstract of the contribution: This contribution proposes a new key issue on NWDAF-assisted charging information generation to support differentiated pricing option.

Discussion

This new key issue is to address the use case #x NWDAF-assisted charging information generation to support differentiated pricing option described in TR 23.700-91.

Proposal

Proposal: It is proposed to add a new key issue on NWDAF-assisted charging information generation to support differentiated pricing option.

The following changes are proposed for FS_eNA TR 23.700-91.

* * * * Start of Changes * * * *

5.2.X	Key issue #X: < NWDAF-assisted charging information generation >

5.2.X.1	Description

As described in SA5 TS 28.201, slice load level analytics could be consumed by CHF for the network slice charging purpose. The architectural between NWDAF and CHF/BSS system is depicted in the following figure.

Figure 1: The relationship between NWDAF and CHF

In addition to per slice charging based on analytics provided by NWDAF, NWDAF can brings more values  for the 5G services, such as VR/AR, gaming, autonomous cars, industry automation and e-health It would be beneficial for the operators if they can take advantage of the data analytics provided by NWDAF.. For example,  charging based on service MOS, which is the combination of bandwidth, latency and packet loss, RSRP, etc.

Moreover, for the VIP users, NWDAF should be able to provide real-time or near real-time service MOS to generate the charging information and perform the corresponding actions (e.g. may provide warning  to the VIP user about poor Service Experience to avoid or reduce complaint from the VIP users as much as possible.

This key issue describes analytics information that may be provided by NWDAF to assisted charging information generation to support differentiated pricing and billing option by CHF/BSS system.

Solutions to this key issue are expected to address the following aspect:

- Which kind of data analytics should be provided by the NWDAF  to its charging related service consumer?

- What kind of information the NWDAF would need to collect, and from which NFs, in order to build analytics related to charging?

NOTE: The specification on the information exchanged between the NWDAF and the CHF needs to be coordinated with SA5.

* * * * End of Changes * * * *