SA WG2 Meeting #S2-136	S2-1910935

18 - 22 November, 2019, Reno, Nevada, USA

Source:	Thales S.A

Title:	Evaluation of Solutions and Conclusions for KI#10

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_5GSAT_ARCH/Rel.16

Abstract of the contribution: Evaluation of Solutions and Conclusions for KI#10

# Discussion

- Solution X provides a solution based on the location of the UE and on the geographical definition of the cells.

- It is proposed to elaborate clause 7.1 on the basis of the evaluation of this function.

- It is proposed to reflect these aspects in the conclusions part of the TR 23.737 in section 8.10 as shown below.

# Proposal

It is proposed to modify clause  7.10 & 8.10 as follows:

***** The 1st change *****

## 7.10	Evaluation of solutions for Key Issue #10: Regulatory services with super-national satellite ground station

1.

For Key Issue #10 - " Key Issue #10: Regulatory services with super-national satellite ground station " One candidate solutions has been proposed, Candidate solutions #X serves the purpose of mitigating the large coverage areas of satellite access.Candidate Solution #X requires that UEs are capable of locating themselves, and that cells are geographically defined. This presents also the advantage of not impacting mobility management procedures.  NG-RAN would have to be capable of geographically managing and handling cells. For NG-RAN embarked in NGSO satellites, NG-RAN would have to be provided their position and to dynamically change the SIB. This Candidate Solution is also applicable Key Issues 6 and 1.  No assumption is imposed on whether the satellite coverage is fixed or not, and on the size of the satellite coverage.

In both cases CellID remains the location information that can be used for operations & management services, for location service and for regulatory services. The size and shape of the satellite cell remains the determining acceptability parameter

***** end of change *****

***** The 2nd change *****

## 8.10 	Conclusion on solutions for Key Issue #10

.

Following the evaluation in of Candidate solutions for  Key Issue #10 “Regulatory services with super-national satellite ground station” in clause 7.10, it is concluded to take Candidate Solution #X as the baseline for normative work on this key issue.

NOTE: 	As this Candidate Solution relies on an approach for which some features  are defined in TSG-RAN WGs, the specification of this solution during the normative phase shall take into account inputs received from TSG-RAN WGs.

***** end of change *****