3GPP TSG-WG SA2 Meeting #136 	S2-1911264

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-1909107)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0058 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of QoS Sustainability Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC, eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some changes are needed in order to be inline with the terminology of other clauses of TS 23.288, to limit the signalling overhead caused by notifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The term “observation period” is changed into “analytics target period”; - The threshold minimum delta (or hysteresis) related to the non-critical direction is introduced to limit unnecessary notifications; - It is specified that the confidence of the analytics is applicable for future time period; - Some editorial corrections are applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminilogy is not inline with TS 23.288 and signalling flooding caused by non-critical notifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 6.9.3, 6.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.9	QoS Sustainability Analytics

### 6.9.1	General

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target periodin the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters:

-	Analytics ID = "QoS Sustainability".

-	Analytics Filter Information:

-	QoS requirements:

-	Standardized 5QI, and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR);

-	For non-standardized QoS Characteristics, the QoS Characteristics attributes PDB and PER and their values (optional).

-	Location information: could be an area or a path of interest. The location information could reflect a list of waypoints.

NOTE:	In this Release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) which is understandable by NWDAF.

-	Analytics target period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested.

-	Threshold(s): The threshold(s) indicate level(s) used for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification when the threshold(s) provided in the analytics subscription are crossed by the expected QoS KPIs. The level(s) relate to value(s) of the QoS KPIs defined in TS 28.554 [10], for the relevant 5QI:

-	for a 5QI of GBR resource type, the Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Threshold(s) refer to the RAN UE Throughput KPI.

Every threshold may include a Minimum Threshold Delta: the acceptable deviation from a threshold in the non-critical direction (i.e. in which the QoS is improving) without triggering reporting to limit the amount of signaling caused by the notifications when the expected KPI is oscillating close to the threshold.

-	S-NSSAI (optional).

The NWDAF collects the corresponding statistics information on the QoS KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow Retainability or the RAN UE Throughput as defined in TS 28.554 [10].

If the Analytics target period refers to the past:

-	The NWDAF verifies whether the triggering conditions for the notification of QoS change statistics are met and if so, generates for the consumer one or more notifications.

-	The analytics feedback contains the information on the location and the time for the QoS change statistics and the threshold(s) that were crossed.

If the Analytics target period is in the future:

-	The NWDAF detects the need for notification about a potential QoS change based on comparing the expected values for the KPI of the target 5QI against the threshold(s) provided by the consumer in any cell in the requested area for the requested Analytics target period. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

-	The analytics feedback contains the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed.

# * * * * Second change * * * *

### 6.9.3	Output analytics

The NWDAF outputs the QoS Sustainability analytics. Depending on the Analytics target period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.9.3-1.

Table 6.9.3-1: "QoS Sustainability" Analytics

| Information | Description |
| --- | --- |
| Analytics |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Threshold(s) | The threshold(s) that are met or exceeded by the statistics value or the expected value of the QoS KPI. |
| >Confidence | Confidence of the analytics (for future Applicable Time Period only). |

NOTE 1:	The meaning of Confidence is based on the SLA, i.e. the consumer has to understand the meaning of the different values of Confidence.

NOTE 2:	The Analytics can contain multiple sets of the above information in case the location information reflected a list of waypoints.

# * * * * Third change * * * *

### 6.9.4	Procedures

Figure 6.9.4-1 depicts a procedure for "QoS Sustainability" analytics provided by NWDAF.

Figure 6.9.4-1: "QoS Sustainability" analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period" when requesting QoS Sustainability analytics for monitoring for a path of interest.

2.	The NWDAF collects the data specified in clause 6.9.2 from the OAM.

3.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the threshold(s) provided by consumer in any cell over the requested Analytics target period.

4.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

# * * * * End of changes * * * *