3GPP TSG-WG SA2 Meeting #136 	S2-1911524

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-190xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3563 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR TS 23.401 Support N7 interface for GERAN/UTRAN access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, China Unicom, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current release, both Gx interface and N7 interface have been defined for policy and charging control of GERAN/UTRAN/E-UTRAN and E-UTRAN/NR combination respectively. While operators may want to simplify the network deployment and reduce the operational complexity of PGW-C/GGSN/SMF and the interface between PGW-C/GGSN/SMF and PCF/PCRF. With such consideration, the PGW needs to support N7 interface to interact with PCF, and support the functionalities required for GERAN/UTRAN access, e.g. the mapping from EPS QoS information to R99 QoS information, as defined in Annex E. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An annex is added to describe the support of N7 interface for GERAN/UTRAN access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PGW/GGSN/SMF has to support both Gx and N7 interfaces for policy control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex YY (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex YY (normative):
Support of N7 interface for Mobility between GERAN/UTRAN and E-UTRAN

During the mobility between GERAN/UTRAN and E-UTRAN, the anchored PGW may support N7 interface, as defined in the TS 23.501 [83], TS 23.502 [84] and TS 23.503 [88], to interact with PCF for Policy and Charging Control.

In case the UE accesses via GERAN/UTRAN, the PGW performs as defined in Annex ZZ of the TS 23.060 [7].

In case the UE accesses via E-UTRAN, the PGW performs as defined in clause 4.11.0a.2 of the TS 23.502 [84].

# * * * * End of changes * * * *