3GPP TSG-SA WG2 Meeting #136	S2-1911624

# Reno, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0021 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve editor’s note on PC5 Link-AMBR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.287 includes editor’s note below: Editor's Note: Whether PC5 LINK-AMBR is required and its relationship with UE PC5 AMBR will be reviewed based on RAN design of PC5. As per current specification, different interpretations can be possible regarding the usage of PC5 LINK-AMBR. - PC5 LINK-AMBR applied to each PC5 unicast link Each PC5 unicast link is associated with the PC5 LINK-AMBR. Aggregate bit rate of a PC5 unicast link shall not exceed PC5 LINK-AMBR. - PC5 LINK-AMBR applied to all PC5 unicast link PC5 LINK-AMBR is in total for all PC5 unicast links. Aggregate bit rate of all PC5 unicast links shall not exceed PC5 LINK-AMBR. Authors of this CR propose to go for the first interpretation above, i.e. applying PC5 LINK-AMBR to each PC5 unicast link. In addition, this CR also clarifies the relationship between PC5 LINK-AMBR and UE-PC5-AMBR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN and clarify PC5 LINK-AMBR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st change ***

5.4.2.3	PC5 Link Aggregated Bit Rates

A PC5 unicast link is associated with the following aggregate rate limit QoS parameter:

-	per link Aggregate Maximum Bit Rate (PC5 LINK-AMBR).

The PC5 LINK-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS Flows with a peer UE over PC5 unicast link.  The PC5 LINK-AMBR is measured over an AMBR averaging window which is a standardized value. The PC5 LINK-AMBR is not applicable to GBR QoS Flows. A PC5 unicast link is associated with the PC5 LINK-AMBR. Aggregate bit rate of one PC5 unicast link shall not exceed PC5 LINK-AMBR. The UE PC5 AMBR shall be set to the sum of the aggregate maximum bit rate of all types of communication (i.e. unicast, groupcast and broadcast modes) over PC5 reference point.

NOTE:	The AMBR averaging window is only applied to PC5 LINK-AMBR measurement.

*** End of the changes ***