3GPP TSG-SA WG Meeting # 136	S2-1911635

# Reno, USA, 18-22 November 2019	was S2-1910348

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1833 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | changing IAB-MT to IAB-UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Vodafone, Qualcomm Inc, Mediatek, AT&T, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The term MT has a precise meaning in 3GPP, and it excludes the SIM/USIM. Hence the use of “MT” within the abbreviation “IAB-MT” is misleading, see also paper S2-1909508. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The term IAB-MT is replaced by IAB-UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing terminology, need to update all specs to say if something applies to IAB-MT in addition to UE! |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.1, 5.35.2, 5.35.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3 changes: - “ME” was used in the coversheet instead of “MT” (summary of changes) - Clauses affected (unchanged clauses needed to be removed) - Changes on changes removed |  |  |  |  |  |  |  |  |

Start of changes

## 5.35	Support for Integrated access and backhaul (IAB)

### 5.35.1	IAB architecture and functional entities

Integrated access and backhaul (IAB) enables wireless in-band and out-of-band relaying of NR Uu access traffic via NR Uu backhaul links. The Uu backhaul links can exist between the IAB-node and

- a gNB referred to as IAB-donor; or

- another IAB-node.

The part of the IAB node that supports the Uu interface towards the IAB-donor or another parent IAB-node (and thus manages the backhaul connectivity with the PLMN it is registered with) is referred to as an IAB-UE.

At high level, IAB has the following characteristics:

-	IAB uses the CU/DU architecture defined in TS 38.401 [42], and the IAB operation via F1 (between IAB-donor and IAB-node) is invisible to the 5GC;

-	IAB performs relaying at layer-2, and therefore does not require a local UPF;

-	IAB supports multi-hop backhauling;

-	IAB supports dynamic topology update, i.e. the IAB-node can change the parent node, e.g. another IAB-node, or the IAB-donor, during operation, for example in response to backhaul link failure or blockage.

Figure 5.35.1-1 shows the IAB reference architecture with two backhaul hops, when connected to 5GC.

Figure 5.35.1-1: IAB architecture for 5GS

Editor's note:	This figure may be updated based on progress on IAB in RAN WGs.

The gNB-DU in the IAB-node is responsible for providing NR Uu access to UEs and child IAB-nodes. The corresponding gNB-CU function resides on the IAB-donor gNB, which controls IAB-node gNB-DU via the F1 interface. IAB-node appears as a normal gNB to UEs and other IAB-nodes and allows them to connect to the 5GC.

The IAB-UE function behaves as a UE, and reuses UE procedures to connect to:

-	the gNB-DU on a parent IAB-node or IAB-donor for access and backhauling;

-	the gNB-CU on the IAB-donor via RRC for control of the access and backhaul link;

-	5GC, e.g. AMF, via NAS;

-	OAM system via a PDU session or PDN connection (based on implementation).

Editor's note:	It is FFS if new 5QI/QCI needs to be defined for the OAM traffic.

The IAB-UE can connect to 5GC over NR (SA mode) or connect to EPC (EN-DC mode). The UE connecting to the IAB-node can operate in the same or different modes than the IAB-node as defined in TS 38.401 [42].

### 5.35.2	5G System enhancements to support IAB

In IAB operation, the IAB-UE interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401 [42].

Editor's note: The following are the expected system enhancements to assist the development of the CRs for IAB. It will be revised and updated based on RAN WG's conclusion.

For the IAB-UE operation, the existing UE authentication methods as defined in TS 33.501 [29] applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

Editor's note:	Security aspect is being studied by SA WG3, and the above will be revised and aligned after the conclusion of the study.

The following aspects are enhanced to support the IAB operation:

-	the Registration procedure as defined in TS 23.502 [3] clause 4.2.2.2 is enhanced to indicate IAB-node's capability to the AMF;

Editor's note:	Whether this indication is provided by the IAB-node at RRC level will be determined by RAN WGs.

-	the UE Subscription data as defined in TS 23.502 [3] clause 5.2.3 is enhanced to include the authorization information for the IAB operation;

-	Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information;

-	UE Context setup/modification procedure is enhanced to provide IAB authorized indication to NG-RAN.

After registered to the 5G system, the IAB-node remains in CM-CONNECTED state. In the case of radio link failure, the IAB-UE uses existing UE procedure to restore the connection with the network. The IAB-UE uses Deregistration Procedure as defined in TS 23.502 [3] clause 4.2.2.3 to disconnect from the network.

### 5.35.3	Data handling and QoS support with IAB

Figure 5.35.3-1 shows the user-plane protocol stack for the UE defined when served by the IAB-nodes. Figure 5.35.3-2 shows the control-plane protocol stack for the UE.

Editor's note:	The figures may be updated or removed when they are formally documented in RAN specifications. For example, the lower layer aspect, e.g. the F1 aspect and BAP and RLC/PHY can be replaced with "Lower Layer", reference to RAN specifications can be added.

Figure 5.35.3-1: IAB user-plane protocol stack

Figure 5.35.3-2: IAB control-plane protocol stack

QoS management for IAB can remain transparent to the 5GC. In case NG-RAN cannot meet a QoS requirement for a QoS flow to IAB-related resource constraints, the NG-RAN can reject the request using procedures defined in TS 23.502 [3].

Editor's note:	Support for F1 security protection is mandated by TS 33.501 [29]. For IAB, F1 security protection may be provided via NDS/IPsec. SA WG3 currently evaluates security procedures for IAB in a separate IAB SI/WI. Based on this SI/WI, IAB security may change. But, this is not expected to have SA WG2 impacts.

The IAB-UE function can establish a PDU session or PDN connection, e.g. for OAM purpose (protocol stack not shown here). In that case, the IAB-UE obtains an IP address/prefix from the core network using normal UE procedures. The IAB-UE's IP address is different from that of the IAB-node's gNB DU IP address.

End of changes