3GPP TSG-WG SA2 Meeting #135 	S2-1912253

# Reno, NV, USA, November 18 – 22, 2019	(revision of S2-1911183)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0071 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to UE related analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 23.501 Clause 5.6.7.1, When the AF request targets an individual UE, the GPSI is provided within the AF request and the GPSI is mapped to SUPI according to the subscription information received from UDM. The SUPI is used only inside 3GPP system, and its privacy is specified in TS 33.501. Further only the GPSI is needed for addressing a 3GPP subscription in different data networks outside of the 3GPP system. The 3GPP system stores within the subscription data the association between the GPSI and the corresponding SUPI. Also it is specified in TS 23.501 that the groups of UEs are identified by an External Group Identifier as defined when the AF interacts via the NEF, or Internal-Group Identifier (see clause 5.9.7) when the AF interacts directly with the PCF. Thus, it is reasonable to clarify that in case of interactions between AF and NWDAF via NEF, the Target of Analytics Reporting can be a GPSI or an External Group Identifier when the interaction is between the AF and NEF that is mapped to a SUPI or an Internal Group Identifier when the interaction is between the NEF and the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note clarifying the use of identifiers in case of interactions between AF and NWDAF via NEF is added in clase 6.7.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncertainties related to the use of identifiers in case of interactions between AF and NWDAF via NEF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.7.4	Expected UE behavioural parameters related network data analytics

#### 6.7.4.1	General

The clause 6.7.4 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, UDM, AF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "UE Mobility" or "UE Communication".

-	The Target of Analytics Reporting can be any UE or an Internal Group Identifier.

NOTE: In case of interactions between AF and NWDAF via NEF, the Target of Analytics Reporting can be a GPSI or an External Group Identifier when the interaction is between the AF and NEF that is mapped to a SUPI or an Internal Group Identifier when the interaction is between the NEF and the NWDAF.

-	An Analytics target period, which indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.4.3.

# * * * * End of change * * * *