3GPP TSG-SAWG2 Meeting #137E	S2-2001849

24 – 27 February 2020, Electronic Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2138 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of direct vs indirect communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Now the NF service consumer is configured to either use direct or indirect communication, being unique for all the producers a consumer may contact. However, the communication to the NRF may be direct (mode C) while for the rest of the producers may be indirect. Then, the consumer needs to take into account the producer to determine whether direct or indirect communication should be used. More generically, the selection of the communication mode should be more flexible. For example, it is important that the communication within the same site does not require indirect communication, while intersite it may be preferrable. Also, this flexibility should apply for all outgoing message request from an NF. If only for NF consumers, then notifications will be excluded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is added to the possibility to configure the communication mode per NF service consumer, that some rules may be evaluated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to properly configured an NF to use indirect mode but as well have a direct communication to the NRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START CHANGES ***

## 7.1	Network Function Service Framework

### 7.1.1	General

Service Framework functionalities include e.g. service registration/de-registration, consumer authorization, service discovery, and inter service communication, which include selection and message passing. Four communication options are listed in Annex E and can all co-exist within one and the same network.

An NF service is one type of capability exposed by an NF (NF Service Producer) to other authorized NF (NF Service Consumer) through a service-based interface. A Network Function may expose one or more NF services. Following are criteria for specifying NF services:

-	NF services are derived from the system procedures that describe end-to-end functionality, where applicable (see  TS 23.502 [3], Annex B drafting rules). Services may also be defined based on information flows from other 3GPP specifications.

-	System procedures can be described by a sequence of NF service invocations.

NF services may communicate directly between NF Service consumers and NF Service Producers, or indirectly via an SCP. Direct and Indirect Communication are illustrated in Figure 7.1.1-1. For more information, see Annex E and clauses 6.3.1 and 7.1.2. Whether a NF Service Consumer uses Direct Communication or Indirect Communication by using an SCP is based on configuration of the NF . For each message to be sent, the NF may, based on local configuration, determine whether the message shall be sent via an SCP.

NOTE 1:	The configuration in the NF when to send via an SCP is not specified but can be a set of rules, e.g. not use SCP in a certain location, use SCP for a set of services, etc.

NOTE 2:	The SCP can be deployed in a distributed manner.

In Direct Communication, the NF Service consumer performs discovery of the target NF Service producer by local configuration or via NRF. The NF Service consumer communicates with the target NF Service producer directly.

In Indirect Communication, the NF Service consumer communicates with the target NF Service producer via a SCP. The NF Service consumer may be configured to perform discovery of the target NF Service producer directly, or delegate the discovery of the target NF Service Producer to the SCP used for Indirect Communication. In the latter case, the SCP uses the parameters provided by NF Service consumer to perform discovery and/or selection of the target NF Service producer. The SCP address may be locally configured in NF Service consumer.

Figure 7.1.1-1: NF/NF service inter communication

*** END CHANGE ***