3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2001865r02

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0089 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | - | ME | - | Radio Access Network | - | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of QoS sustainability analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It should be clarified that V2X AS should assign parameters in the request for notification in such a way that there is enough time when the analytics is received to implement an action (a countermeasure) and the action needs to be completed before the time when the QoS change happens. - Editorial changes are needed to be in line with QoS sustainability analytics of TS 23.288 (clause 6.9) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A note to the step 2 of procedure for notification on QoS Sustainability Analytics is added to clarify that the parameters in the request on analytics should be assigned by V2X AS in such a way that on receiving the response to complete V2X application adjustment before the QoS change happens. - Editorial changes related to notification on QoS Sustainability Analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Out-of-date and useless notification on QoS sustainability analytics can be delivered to the V2X AS and preventing it to take countermeasures in time with V2X application. Description of QoS sustainability analytics is not inline with TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5.2.2, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.4.5.2.2	Functional description

The potential QoS change to assist application adjustment is based on the notification of QoS Sustainability Analytics defined in clause 6.9 of TS 23.288 [20]. The V2X Application Server acting as an Application Function communicates with the NEF which corresponds to the NF consumer in clause 6.9.4 of TS 23.288 [20].

The V2X Application Server can either subscribe to notifications from the NEF (i.e., a Subscribe-Notify model) or request a single notification from the NEF (i.e. a Request-Response model). The V2X Application Server request contains the following parameters:

-	Analytics ID = "QoS Sustainability";

-	Target of Analytics Reporting: "any UE";

-	Analytics Filter Information:

-	Optional maximum number of results;

-	QoS requirements:

-	5QI (standardized or pre-configured), and applicable additional QoS parameters and the corresponding values (conditional, i.e., it is needed for GBR 5QIs to know the GFBR); or

-	the QoS Characteristics attributes including Resource Type, PDB and PER and their values ;

NOTE:	The V2X Application Server could provide the 5QIs as input based on SLA with the operator which authorizes the V2X Application Server to use certain 5QIs and to use the QoS Sustainability Analytics only for them.

-	Location information (the details are specified in TS 23.288 [20] clause 6.9.1);

-	S-NSSAI (optional, if available in the V2X Application Server).

-	Analytics target period (the details are specified in TS 23.288 [20] clause 6.9.1);

-	Reporting Threshold(s) (the details are specified in TS 23.288 [20] clause 6.9.1);

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

# * * * * Second change * * * *

## 6.4	Procedures for V2X communication over Uu reference point

### 6.4.1	Procedure for notification on QoS Sustainability Analytics to the V2X Application Server

Figure 6.4.1-1 depicts a procedure for notification on QoS Sustainability Analytics to the V2X Application Server.

Figure 6.4.1-1: Notification on QoS Sustainability Analytics to the V2X Application Server

1.	The V2X Application Sever derives information on V2X service, location information (an area or path of interest),  QoS requirements and reporting threshold(s). The location information can be in the form of geographic coordinates to reflect a list of waypoints.

NOTE 1:	Step 1 is outside 3GPP scope. The V2X Application Server can derive this, for example, based on information provided by the UE(s) or the V2X Application Server uses locally configured information.

2.	The V2X Application Server subscribes to or requests notification on QoS Sustainability Analytics provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [20]. The parameters included in the request are described in clause 5.4.5.2.2.

The V2X Application Server may include multiple sets of parameters in order to provide different combinations of "Location information" and "Time information" when requesting notification on QoS Sustainability Analytics. That is, one request may include multiple sets of parameters to reflect different timescales that may assist the V2X Application Server for monitoring of a path of interest.

NOTE 2:	It is assumed that the "Time when analytics information is needed" parameter (see TS 23.288 [20]) is large enough to allow the V2X application adjustment to complete before the potential QoS change happens.

NOTE 3:	The request made by the V2X Application Server does not have to be per UE. In order to make a request for multiple UEs, the V2X Application Server can perform mapping between individual UE path requests and subscription to notification per path/road segment. Whether a request is generated for a UE or multiple UEs, and how to set the parameters included in the request and triggers for such request are up to the V2X Application Server implementation and outside of the scope of 3GPP.

3.	When the NEF receives the request from the V2X Application Server, the NEF interacts with the NWDAF as described in the procedure in clause 6.9.4 of TS 23.288 [20]. The NEF shall map the QoS Characteristics attributes including Resource Type, PDB and PER received from the V2X Application Server to the most appropriate 5QI that is supported by the network if the V2X application server does not provide 5QI. If 5QI is provided by the V2X Application Server, NEF does not perform such mapping. The NEF may map other parameters included in the request from the V2X Application Server to information used by the 3GPP system.

4.	If the NEF receives the response from the NWDAF, the NEF notifies the V2X Application Server with the analytics information on QoS Sustainability Analytics by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [20].

5.	V2X application adjustment may take place.

NOTE 4:	Step 5 is outside 3GPP scope. V2X application adjustment can take place at UE and/or V2X Application Server, e.g. adjust inter-vehicle gap, change video codec parameters, etc. For the case that the V2X Application Server uses the request for multiple UEs in step 2, the V2X Application Server can perform the V2X application adjustment for every UE corresponding to the request.

# * * * * End of changes * * * *