3GPP TSG-SA2 Meeting #137E	S2-2001880

# Feb 24-27, 2020, Electronic Meeting	(revision of S2-2001267, 0282)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1977 | rev | 2 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Emergency service at EPC-5GC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2020-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#136, S-NSSAI setting in AMF is clarified in 23.501CR1984 (S2-1912193, Rel-15) and CR1985 (S2-1912194, Rel-16), however the necessary update is still missing in the procedures. For the following procedures, considering that AMF may or may not be able to get the S-NSSAI from the HSS+UDM, it’s proposed that the AMF use the S-NSSAI locally configured in the Emergency Configuration Data to simplify. - EPS to 5GS mobility without N26; - Handover from EPS to 5GC/N3IWF; - Handover from EPC/ePDG to 5GS In addition, how 5GC IWK indication is set in EPC and how EPC IWK indication is set in 5GC for emergency PDU Session is also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 The AMF uses the S-NSSAI in the Emergency Configuration Data in the following mobility procedures: - EPS to 5GS mobility without N26; - Handover from EPS to 5GC/N3IWF; - Handover from EPC/ePDG to 5GS. (For EPS to 5GS mobility with N26, PGW-C+SMF returns S-NSSAI to AMF as for non-emergency PDU Session). #2 At emergency PDN connection setup in EPC: 1-1 Clarify that MME or ePDG sets the 5GC IWK indication based on UE’s 5GC NAS capability and local configuration. 1-2 Clarify the PGW-C+SMF handles S-NSSAI as follows: Only one S-NSSAI shall be configured in PGW-C+SMF for the emergency APN; No check against the UDM is needed (UE may not be authenticated) S-NSSAI is not sent to UE in PCO. 1-3 Clarify that MME or ePDG may select combined PGW-C+SMF based on Emergency Configuration Data configured in MME or ePDG, which may contain statically configured emergency PGW-C+SMF. #3 At emergency PDU Session setup in 5GC, The AMF sets EPC IWK indication based on UE’s S1 mode support and local configuration. Rev2: Clause 4.11.5.3, correct the description for PDU Session with Request type “initial emergency request”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency service not properly specified causing potential failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Changes * * * *

#### 4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. If the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values.

During establishment of emergency PDN connection,

- The PGW-C+SMF is to be derived from the emergency APN or to be statically configured in the Emergency Configuration Data in MME;

- If the emergency APN is valid for more than one S-NSSAI, PGW-C+SMF selects an S-NSSAI based on local policy.

* * * * End of Changes * * * *